Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE838 |
| Course Title | SOC Design |
| Semester/Year | W2026 |
| Instructor | Gul N. Khan |

| Lab/Tutorial Report NO. | Lab #1 Report |
| --- | --- |

| Report Title | Lab #1: COE838: Systems-on-Chip Design LAB-1 |
| --- | --- |

| Section No. | 03 |
| --- | --- |
| Submission Date | January 27, 2025 |
| Due Date | January 27, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Abstract

This lab project will design and simulate an ALU (arithmetic logic unit) integrated with a barrel shifter to conduct basic 8-bit addition/subtraction operations, with (optional) pre-processing of one operand through left or right shifting. The design was created using modular SystemC components, which was a barrel shifter for data preprocessing and an ALU for arithmetic operations. Using a SystemC testbench and GTKWave, the design was verified and the system operated smoothly. The completed project properly sequences between the barrel shifter and ALU, as well as completes correct arithmetic functionality. This lab confirms validity in the use of event-driven modeling in SystemC for simple System-on-Chip designs.

## Introduction

SystemC is a C++ based hardware based description and simulation library which is used for creating complex digital systems. SystemC presents hardware models and communication channels in the form of high-level programming languages. System-on-Chip architecture commonly consists of key modules such as the ALU and barrelshift in real life hardware. This lab is to gain a familiarity with basic SystemC modules and how to implement them, in order to gain a better understanding of fundamental SoC design.

## Methodology

The system has two SystemC modules:

- Barrel Shifter: An input “B” shifts left/right by a specified number, or passes the input unchanged (when disabled)

- ALU: completes addition/subtraction of input A on the processed “B” value

The barrel shifter will execute before the ALU, which will ensure proper sequencing. Control signals (op, en, l_r, and shift_amt) determine the overall system behavior.

## Test Cases/Waveforms (Results)

This design was tested using many scenarios to verify functionality. Signal behaviour was analyzed with GTKWave.

- Test Case #1:

en = 0

op = 1

Input B passes through unchanged and is added to A.

Figure #1: Test Case #1 Waveform (ALU)

- Test Case #2:

en = 1

l_r = 0

op = 0

Input B is shifted left by shift_amt before being added to A.

Figure #2: Test Case #2 Waveform (Barrelflop)

- Test Case #3:

en = 1

L_r = 0

op = 0

Input B is shifted right by shift_amt and then subtracted from A.

Figure #3: Test Case #3 Waveform (Barrelshift)

- Test Case #4:

en = 0

op = 0

Input B passes through unchanged and is subtracted from A.

Figure #4: Test Case #4 Waveform (Flipflop)

## Challenges, Outstanding Work, and Future Improvements

The main challenge while designing the project, was to ensure that the ALU executes after the barrelshift to maintain correct data flow. This requires careful attention to detail and correctness in signal connections and sensitivity lists within the SystemC modules.

Future improvements made to the project can include allowing the ALU to support additional arithmetic and logical operations such as multiplication and comparisons. Further improvement could also involve pipelining the barrel shifter and ALU stages to model higher-performance SoC architectures.

## Discussion/Conclusion

The SystemC-based ALU and barrel shifter were properly created and work well. The outputs verified the ALU and barrel shifter through waveform analysis. Each test’s output behaviour was expected, and confirms correct operations as well as satisfies lab requirements. The combined system shows correct sequencing between barrel shifter and ALU modules, validating event-driven SystemC modeling.

## References

## References

## Toronto Metropolitan University. (n.d.). COE838: Systems-on-Chip Design LAB-1 [PDF]. D2L. https://courses.torontomu.ca/d2l/le/content/1120983/viewContent/6746565/View

## Appendix

### alu.cpp

#include "alu.h" void make_alu() { sc_trace_file *tf; // Create VCD trace file, can use GTKWave to view waveforms sc_signal<sc_int<8>> a_in, b_in, out; sc_signal<sc_uint<1>> op; sc_signal<bool> en; sc_clock clk("clk", 10, SC_NS, 0.5); // Create Device Under Test (DUT) alu alu_DUT("alu"); alu_DUT.clk(clk); alu_DUT.op(op); alu_DUT.a_in(a_in); alu_DUT.b_in(b_in); alu_DUT.out(out); tf = sc_create_vcd_trace_file("trace_file"); tf->set_time_unit(1, SC_NS); sc_trace(tf, clk, "clk"); sc_trace(tf, op, "op"); sc_trace(tf, a_in, "a_in"); sc_trace(tf, b_in, "b_in"); sc_trace(tf, out, "out"); cout << "\nExecuting ALU, check .vcd file in GTKWave\n" << endl; // Start testbench, initialize first op.write(0); a_in.write(0); b_in.write(0); sc_start(10, SC_NS); // Test addition ¦ 2 + 3 = 5 op.write(1); a_in.write(2); b_in.write(3); sc_start(10, SC_NS); // Test subtraction ¦ 4 - 1 = 3 op.write(0); a_in.write(4); b_in.write(1); sc_start(10, SC_NS); // Test subtraction with negative difference ¦ 5 - 7 = -2 op.write(0); a_in.write(5); b_in.write(7); sc_start(10, SC_NS); sc_close_vcd_trace_file(tf); }

### Sc_main.cpp

#include <systemc.h> #include "alu.h" #include "barrelshift.h" int sc_main(int argc, char* argv[]) { //make_alu(); //make_barrel(); sc_trace_file *tf; // Create VCD trace file, can use GTKWave to view waveforms // Initialize signals sc_signal<sc_int<8>> a_in, b_in, bs_out, alu_out; sc_signal<sc_uint<1>> op; sc_signal<bool> enable, left_right; sc_signal<sc_uint<4>> shift_amt; sc_clock clk("clk", 10, SC_NS, 0.5); // Create Device Under Test (DUT) barrel_shift bs_DUT("bs"); bs_DUT.clk(clk); bs_DUT.enable(enable); bs_DUT.left_right(left_right); bs_DUT.input(b_in); bs_DUT.output(bs_out); bs_DUT.shift_amt(shift_amt); // sc_in<bool> clk; // sc_in<bool> enable; // sc_in<bool> left_right; // sc_inout<sc_int<8>> input, output; alu alu_DUT("alu"); alu_DUT.clk(clk); alu_DUT.op(op); alu_DUT.a_in(a_in); alu_DUT.b_in(bs_out); alu_DUT.out(alu_out); tf = sc_create_vcd_trace_file("trace_file"); tf->set_time_unit(1, SC_NS); sc_trace(tf, clk, "clk"); sc_trace(tf, op, "op"); sc_trace(tf, a_in, "a_in"); sc_trace(tf, b_in, "b_in"); sc_trace(tf, enable, "enable"); sc_trace(tf, left_right, "left0_right1"); sc_trace(tf, shift_amt, "shift_amt"); sc_trace(tf, alu_out, "alu_out"); cout << "\nExecuting ALU, check .vcd file in GTKWave\n" << endl; // Start testbench, initialize first // op ¦ 0 = sub, 1 = add // a_in ¦ 8-bit integer // b_in ¦ 8-bit integer // enable ¦ 0 = barrelshifter off, 1 = barrelshifter on // left_right ¦ 0 = shift left, 1 = shift right op.write(0); a_in.write(0); b_in.write(2); enable.write(1); left_right.write(0); shift_amt.write(1); sc_start(10, SC_NS); // Test addition with left shift ¦ 2 + (3<<1) = 8 op.write(1); a_in.write(2); b_in.write(3); enable.write(1); left_right.write(0); shift_amt.write(1); sc_start(10, SC_NS); // Test subtraction with right shift ¦ 9 - (8>>2) = 7 op.write(0); a_in.write(9); b_in.write(8); enable.write(1); left_right.write(1); shift_amt.write(2); sc_start(10, SC_NS); // Test above case with enable = 0 op.write(0); a_in.write(9); b_in.write(8); enable.write(0); left_right.write(1); shift_amt.write(2); sc_start(10, SC_NS); // Test above case with shift_amt = 0 op.write(0); a_in.write(9); b_in.write(8); enable.write(1); left_right.write(1); shift_amt.write(0); sc_start(10, SC_NS); // Test addition with right shift 8 ¦ 7 + (96>>8) = 7 op.write(1); a_in.write(7); b_in.write(96); enable.write(1); left_right.write(1); shift_amt.write(8); sc_start(10, SC_NS); // Test addition with bs disabled ¦ 14 + 86 = 100 op.write(1); a_in.write(14); b_in.write(86); enable.write(0); left_right.write(1); shift_amt.write(8); sc_start(10, SC_NS); sc_close_vcd_trace_file(tf); return 0; }

### Barrelshift.cpp

#include "barrelshift.h" void barrel_shift :: barrel_method(){ sc_int<8> data; int i; if(enable.read() == 1){ data = output.read(); if(left_right.read() == 0){ //shift left cout << "[left] Data = " << data.to_string(SC_BIN) << endl; for(i = 7; i > 0; i--) data[i] = output.read()[i-1]; data[0] = l_in.read(); output.write(data); }else if(left_right.read() == 1){ cout << "[right] Data = " << data.to_string(SC_BIN) << endl; for(i = 0; i < 7; i++)//shift right data[i] = output.read()[i+1]; data[7] = r_in.read(); output.write(data); } } else cout << "Not enabled "<<endl; } void make_barrel(){ sc_trace_file *tf; // Create VCD file for tracing sc_signal<bool> left_in, right_in; //Declare signals sc_signal<bool> en, left_right; sc_signal<sc_int<8> >data_out; sc_clock clk("clk",10,SC_NS,0.5); //Create a clock signal barrel_shift barrel("barrelshift"); //Create testbench Device Under Test (DUT) barrel.l_in(left_in); // Connect/map the ports to testbench signals barrel.r_in(right_in); barrel.clk(clk); barrel.enable(en); barrel.left_right(left_right); barrel.output(data_out); // Create wave file and trace the signals executing tf = sc_create_vcd_trace_file("trace_file"); tf->set_time_unit(1, SC_NS); sc_trace(tf, clk, "clk"); sc_trace(tf, en, "enable"); sc_trace(tf, left_in, "left_data"); sc_trace(tf, right_in, "right_data"); sc_trace(tf, left_right, "left_right_signal"); sc_trace(tf, data_out, "data_out"); cout << "\nExecuting 1bit barrel shift example... check .vcd produced"<<endl; //start the testbench en.write(0); //initialize left_in.write(0); right_in.write(0); left_right.write(0); sc_start(10, SC_NS); en.write(1); //enable and input left_in.write(1); right_in.write(0); left_right.write(0); sc_start(10, SC_NS); left_in.write(1); right_in.write(0); left_right.write(0); sc_start(10, SC_NS); left_in.write(1); right_in.write(0); left_right.write(0); sc_start(10, SC_NS); left_in.write(1); right_in.write(0); left_right.write(0); sc_start(10, SC_NS); left_in.write(1); right_in.write(0); left_right.write(0); sc_start(10, SC_NS); left_in.write(1); right_in.write(0); left_right.write(0); sc_start(10, SC_NS); left_in.write(1); right_in.write(0); left_right.write(1); sc_start(10, SC_NS); en.write(0); //not enabled and input scenario left_in.write(1); right_in.write(0); left_right.write(1); sc_start(10, SC_NS); en.write(1); //enabled left_in.write(0); right_in.write(1); left_right.write(1); sc_start(10, SC_NS); left_in.write(0); right_in.write(1); left_right.write(1); sc_start(10, SC_NS); left_in.write(0); right_in.write(1); left_right.write(1); sc_start(10, SC_NS); left_in.write(0); right_in.write(1); left_right.write(1); sc_start(10, SC_NS); left_in.write(0); right_in.write(1); left_right.write(1); sc_start(10, SC_NS); sc_close_vcd_trace_file(tf); }