Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE838 |
| Course Title | SOC Design |
| Semester/Year | W2026 |
| Instructor | Gul N. Khan |

| Lab/Tutorial Report NO. | Interim Report |
| --- | --- |

| Report Title | NoC - Interim Report |
| --- | --- |

| Section No. | 03 |
| --- | --- |
| Submission Date | March 24rd, 2025 |
| Due Date | March 24th, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

NoC – Interim Report

Shai Mourad

Student ID: 501170415

Department of Mechanical, Industrial, and Mechatronics Engineering

Toronto Metropolitan University

Course: COE838 – SoC Design

Instructor: Gul N. Khan

Abstract – This project presents a creation and simulation, using SystemC, of a NoC (Network-on-Chip). Architecture of a wormhole-routed NoC containing a source, router, and sink module is created and analyzed. A simulation of a  4x4 mesh topology is done and packet transmission between source and destination modules is observed. The lab project demonstrates proper packet receiving and delivering with the average packet delay at 15 ns. This lays the foundation for further optimization of NoC architecture.

## I. Introduction

When SoC (System-on-Chip) designs become more complex, traditional bus-based communication becomes more inefficient as congestion and limited scalability ensue. Network-on-Chip provides a scalable and efficient communication outline by enabling packet based data transfer between cores.

This project uses a SystemC-based NoC to show communication between processing elements. The design uses source modules to generate packets, routers that complete switching/routing, as well as sink modules that receive the packets. This interim work analyzes the architecture while observing the created packet transmission and performance.

## II. Theory

NoC replaces shared buses with routers and links. This allows IP cores to communicate in parallel while improving throughput, scalability, and modularity.

Data transmission is done via packets, which are divided into the following flits (flow control units):

- Header flit: Contains routing information (source and destination IDs)

- Payload flits: Carry actual data

- Tail flit: Indicates the end of the packet

This project also uses wormhole routing where the header flit establishes the path, subsequent flits follow the same route, and buffers store small portions of the packets [1].

## III. System Design

### A. Source Module
A source module generates packets and then injects them into the network

The key operations of the source module:

- Generates synthetic data (e.g., 1001, 1002, …)

- Assigns source ID and destination ID

- Toggles a clock bit to ensure event triggering

- Marks every 5th flit as a tail flit [1]

Source ID = 5

Destination ID = 10

### B. Sink Module

A sink module receives the packets and acknowledges a successful reception.

The key operations of the sink module:

- Detects incoming packets

- Sends acknowledgment to router

- Logs received packet data [1]

C. Router Architecture
The router is composed of FIFO buffers which store incoming filts, an arbiter to decide output port from destinations, and a crossbar switch to route packets to the correct output.

Routing Flow: Packet enters FIFO buffer, request sent to arbiter, arbiter grants access to output port, and crossbar forwards packet.

The implemented router differs from the generic design since only two FIFO buffers are instantiated instead of five and a simplified implementation is used for simulation purposes [1].

Figure 1. 4×4 Mesh NoC Architecture

Figure 2. Router (Internal) Architecture (NOTE: used only 2 FIFOs instead of 5)

Figure 3. Router (Internal) Architecture Reduced to 2 FIF0s

Figure 4. Packet Flow Diagram

Figure 5. Flowchart Diagram

## IV. Results

Figure 6. Terminal Output Screenshot

mp22:/home/student1/smourad/Desktop/COE838/NOC> ./noc.x SystemC 2.3.0-ASI --- Sep 10 2012 16:44:06 Copyright (c) 1996-2012 by all Contributors, ALL RIGHTS RESERVED Enter source ID (0 to 15): 5 Enter Destination ID (0 to 15): 10 ------------------------------------------------------------------------------- 4X4 mesh NOC simulator containing 2 5x5 Wormhole router ------------------------------------------------------------------------------- This is the simulation of a 4x4 Wormhole router. We assume the router has 5 input/output ports, with 4 buffers per input port and each flit has 21 bits width Press "Return" key to start the simulation... WARNING: Default time step is used for VCD tracing. Trace Warning: Traced objects found with name containing [], which may be interpreted by the waveform viewer in unexpected ways. So the [] is automatically replaced by (). t: 125 ns ¦¦ New Pkt: 1001 is sent by source: 5 to Destination: 10 t: 140 ns ¦¦ New Pkt: 1001 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 250 ns ¦¦ New Pkt: 1002 is sent by source: 5 to Destination: 10 t: 265 ns ¦¦ New Pkt: 1002 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 375 ns ¦¦ New Pkt: 1003 is sent by source: 5 to Destination: 10 t: 390 ns ¦¦ New Pkt: 1003 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 500 ns ¦¦ New Pkt: 1004 is sent by source: 5 to Destination: 10 t: 515 ns ¦¦ New Pkt: 1004 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 625 ns ¦¦ New Pkt: 1005 is sent by source: 5 to Destination: 10 t: 640 ns ¦¦ New Pkt: 1005 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 750 ns ¦¦ New Pkt: 1006 is sent by source: 5 to Destination: 10 t: 765 ns ¦¦ New Pkt: 1006 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 875 ns ¦¦ New Pkt: 1007 is sent by source: 5 to Destination: 10 t: 890 ns ¦¦ New Pkt: 1007 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 1 us ¦¦ New Pkt: 1008 is sent by source: 5 to Destination: 10 t: 1015 ns ¦¦ New Pkt: 1008 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 1125 ns ¦¦ New Pkt: 1009 is sent by source: 5 to Destination: 10 t: 1140 ns ¦¦ New Pkt: 1009 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ t: 1250 ns ¦¦ New Pkt: 1010 is sent by source: 5 to Destination: 10 t: 1265 ns ¦¦ New Pkt: 1010 is received from source: 5 by sink: 10 ____________________________________________________________________________________________ ------------------------------------------------------------------------------- End of switch operation... Total number of packets sent: 10 Total number of packets received: 10 ------------------------------------------------------------------------------- Press "Return" key to end the simulation...

Figure 7. Terminal Output (Complete)

Observations:

- All packets were successfully delivered

- Constant delay observed for all packets

- No congestion or contention detected

- System demonstrates stable routing behavior

A. Functional Verification
The simulation confirms a functional NoC system. Packets from the source (Node 5) to the destination (Node 10) were routed through intermediate routers and received without loss. Routing followed an expected XY deterministic path, where packets first traverse in the X-direction before proceeding along the Y-direction. 

The terminal output verifies that the packets reached the destination in proper order, and confirms correctness of the routing logic and buffering mechanisms. No deadlocks or livelocks were detected. 

B. Latency Analysis
The latency observed was approximately constant at 15 ns for all packets. This is due to the use of deterministic XY routing, which ensures a fixed path between a given source–destination pair, absence of network congestion due to single-source traffic, as well as uniform router processing delays across all nodes.

The latency is primarily determined by the number of hops between the source, destination and the per-router processing delay. Due to both remaining constant, the overall delay does not vary (between packets). 

C. Network Behavior Under Limited Traffic/Limitations of Current Results
The results above are limited to a single traffic and therefore do not show the full performance capabilities of the NoC. Although there were no packet queuing delays, high buffer utilizations, or high throughput, these limitations restrict the ability to generalize the observed performance metrics.

E. Discussion and Future Evaluation
To collect a better evaluation, multiple concurrent traffic flows will study congestion effects, measurement will be done for throughput and packet loss under high load, analysis of latency variation will be conducted across different source–destination pairs, as well as completion of an evaluation of alternative routing strategies, if applicable.

## V. Work Progress and Plan

Tasks Completed:

- Understood SystemC NoC architecture

- Simulated 4×4 mesh NoC

- Verified correct packet transmission

- Analyzed timing results

- Computed average packet delay

Tasks to be Completed:

| Component | Status | Issues | Next Step |
| --- | --- | --- | --- |
| NoC Simulation | Completed | Limited traffic model | Add uniform traffic |
| Routing | Working | Basic arbiter | Optimize arbitration |
| Performance | Partial | No stress test | Add congestion scenarios |

Table 1. Summary of Progress

## VI. Conclusion

The interim work successfully models and simulates a 4x4 mesh systemC NoC. The system uses reliable packet delivery with a latency of 15ns. The results show the correctness of the routing mechanism and provide a fantastic foundation for even further improvements such as optimization for topology or traffic.

## VII. References

[1] G. N. Khan, “SystemC Based NoC (Network-on-Chip) Modeling – Course Project,”

Department of Electrical, Computer and Biomedical Engineering, Toronto Metropolitan University,

Toronto, ON, Canada. [Online]. Available: http://www.ecb.torontomu.ca/~courses/coe838/labs/NoC-Project.pdf

## VIII. Appendix

#### main_noc.cpp

// main.cpp #include "systemc.h" #include <iostream> #include <stdlib.h> #include <stdio.h> #include "packet.h" #include "source.h" #include "sink.h" #include "router.h" int sc_main(int argc, char *argv[]) { // Define signals for interfacing modules sc_signal<packet> si_source[16]; sc_signal<packet> si_input[16]; sc_signal<packet> si_zero[64]; sc_signal<packet> si_sink[16]; sc_signal<packet> si_output[64]; // Define acknowledge signals for handshake protocol between modules sc_signal<bool> si_ack_src[64],si_ack_ou[64]; sc_signal<bool> si_ack_sink[16],si_ack_in[64]; sc_signal<bool> si_ack_zero[64]; sc_signal<sc_uint<4> > siid0,siid1,siid2,siid3,siid4,siid5,siid6,siid7,siid8,siid9,siid10,siid11,siid12,siid13,siid14,siid15; sc_signal<sc_uint<4> > scid0,scid1, scid2, scid3,scid4,scid5, scid6, scid7,scid8,scid9, scid10, scid11,scid12,scid13, scid14, scid15; sc_signal<sc_uint<4> > id0,id1, id2, id3, id4,id5, id6, id7,id8,id9, id10, id11,id12,id13, id14, id15; sc_signal<int> scinput; sc_signal<sc_uint<4> > check; sc_signal <packet> sioutput[16]; int i,j; sc_clock s_clock("S_CLOCK", 125, SC_NS, 0.5, 0.0, SC_NS); // source clock //sc_clock s_clock("S_CLOCK", 5, SC_NS, 0.5, 10.0, SC_NS); // source clk = router clk sc_clock r_clock("R_CLOCK", 5, SC_NS, 0.5, 10.0, SC_NS); // router clock sc_clock d_clock("D_CLOCK", 5, SC_NS, 0.5, 10.0, SC_NS); // destination clock // Module instiatiations follow // Note that modules can be connected by hooking up ports // to signals by name or by using a positional notation source source0("source0"); source0(si_source[0], scid0, si_ack_src[0], s_clock, scinput,check); source source1("source1"); source1(si_source[1], scid1, si_ack_src[1], s_clock, scinput,check); source source2("source2"); source2(si_source[2], scid2, si_ack_src[2], s_clock, scinput,check); source source3("source3"); source3(si_source[3], scid3, si_ack_src[3], s_clock, scinput,check); source source4("source4"); source4(si_source[4], scid4, si_ack_src[4], s_clock, scinput,check); source source5("source5"); source5(si_source[5], scid5, si_ack_src[5], s_clock, scinput,check); source source6("source6"); source6(si_source[6], scid6, si_ack_src[6], s_clock, scinput,check); source source7("source7"); source7(si_source[7], scid7, si_ack_src[7], s_clock, scinput,check); source source8("source8"); source8(si_source[8], scid8, si_ack_src[8], s_clock, scinput,check); source source9("source9"); source9(si_source[9], scid9, si_ack_src[9], s_clock, scinput,check); source source10("source10"); source10(si_source[10], scid10, si_ack_src[10], s_clock, scinput,check); source source11("source11"); source11(si_source[11], scid11, si_ack_src[11], s_clock, scinput,check); source source12("source12"); source12(si_source[12], scid12, si_ack_src[12], s_clock, scinput,check); source source13("source13"); source13(si_source[13], scid13, si_ack_src[13], s_clock, scinput,check); source source14("source14"); source14(si_source[14], scid14, si_ack_src[14], s_clock, scinput,check); source source15("source15"); source15(si_source[15], scid15, si_ack_src[15], s_clock, scinput,check); router router0("router0"); // hooking up signals to ports by name router0.in0(si_source[0]); router0.in1(si_output[2]); router0.in2(si_output[12]); router0.in3(si_zero[1]); router0.in4(si_zero[2]); router0.router_id(id0); //router0.router_id(0); router0.out0(si_sink[0]); router0.out2(si_output[0]); router0.out3(si_output[1]); router0.out1(si_zero[3]); router0.out4(si_zero[4]); router0.inack0(si_ack_sink[0]); router0.inack1(si_ack_in[2]); router0.inack2(si_ack_in[12]); router0.inack3(si_ack_zero[1]); router0.inack4(si_ack_zero[2]); router0.outack0(si_ack_src[0]); router0.outack2(si_ack_in[0]); router0.outack3(si_ack_in[1]); router0.outack1(si_ack_zero[3]); router0.outack4(si_ack_zero[4]); router0.rclk(r_clock); router router1("router1"); // hooking up signals to ports by name router1.in0(si_source[1]); router1.in1(si_output[0]); router1.in2(si_output[7]); router1.in3(si_output[16]); router1.in4(si_zero[5]); router1.router_id(id1); //router1.router_id(1); router1.out0(si_sink[1]); router1.out4(si_output[2]); router1.out3(si_output[3]); router1.out2(si_output[4]); router1.out1(si_zero[6]); router1.inack0(si_ack_sink[1]); router1.inack1(si_ack_in[0]); router1.inack2(si_ack_in[7]); router1.inack3(si_ack_in[16]); router1.inack4(si_ack_zero[5]); router1.outack0(si_ack_src[1]); router1.outack4(si_ack_in[2]); router1.outack3(si_ack_in[3]); router1.outack2(si_ack_in[4]); router1.outack1(si_ack_zero[6]); router1.rclk(r_clock); //need 64 code statement router router2("router2"); // hooking up signals to ports by name router2.in0(si_source[2]); router2.in1(si_output[4]); router2.in2(si_output[9]); router2.in3(si_output[17]); router2.in4(si_zero[7]); router2.router_id(id2); //router2.router_id(2); router2.out0(si_sink[2]); router2.out1(si_zero[19]); router2.out2(si_output[5]); router2.out3(si_output[6]); router2.out4(si_output[7]); router2.inack0(si_ack_sink[2]); router2.inack1(si_ack_in[4]); router2.inack2(si_ack_in[9]); router2.inack3(si_ack_in[17]); router2.inack4(si_ack_zero[7]); router2.outack0(si_ack_src[2]); router2.outack1(si_ack_zero[19]); router2.outack2(si_ack_in[5]); router2.outack3(si_ack_in[6]); router2.outack4(si_ack_in[7]); router2.rclk(r_clock); router router3("router3"); // hooking up signals to ports by name router3.in0(si_source[3]); router3.in1(si_output[5]); router3.in2(si_output[21]); router3.in3(si_zero[8]); router3.in4(si_zero[9]); router3.router_id(id3); //router3.router_id(3); router3.out0(si_sink[3]); router3.out3(si_output[8]); router3.out4(si_output[9]); router3.out2(si_zero[20]); router3.out1(si_zero[21]); router3.inack0(si_ack_sink[3]); router3.inack1(si_ack_in[5]); router3.inack2(si_ack_in[21]); router3.inack3(si_ack_zero[8]); router3.inack4(si_ack_zero[9]); router3.outack0(si_ack_src[3]); router3.outack3(si_ack_in[8]); router3.outack4(si_ack_in[9]); router3.outack2(si_ack_zero[20]); router3.outack1(si_ack_zero[21]); router3.rclk(r_clock); router router4("router4"); // hooking up signals to ports by name router4.in0(si_source[4]); router4.in1(si_output[1]); router4.in2(si_output[13]); router4.in3(si_output[26]); router4.in4(si_zero[10]); router4.router_id(id4); //router0.router_id(0); router4.out0(si_sink[4]); router4.out2(si_output[10]); router4.out3(si_output[11]); router4.out1(si_output[12]); router4.out4(si_zero[22]); router4.inack0(si_ack_sink[4]); router4.inack1(si_ack_in[1]); router4.inack2(si_ack_in[13]); router4.inack3(si_ack_in[26]); router4.inack4(si_ack_zero[10]); router4.outack0(si_ack_src[4]); router4.outack2(si_ack_in[10]); router4.outack3(si_ack_in[11]); router4.outack1(si_ack_in[12]); router4.outack4(si_ack_zero[22]); router4.rclk(r_clock); router router5("router5"); // hooking up signals to ports by name router5.in0(si_source[5]); router5.in1(si_output[3]); router5.in2(si_output[10]); router5.in3(si_output[20]); router5.in4(si_output[30]); router5.router_id(id5); //router1.router_id(1); router5.out0(si_sink[5]); router5.out4(si_output[13]); router5.out3(si_output[14]); router5.out2(si_output[15]); router5.out1(si_output[16]); router5.inack0(si_ack_sink[5]); router5.inack1(si_ack_in[3]); router5.inack2(si_ack_in[10]); router5.inack3(si_ack_in[20]); router5.inack4(si_ack_in[30]); router5.outack0(si_ack_src[5]); router5.outack4(si_ack_in[13]); router5.outack3(si_ack_in[14]); router5.outack2(si_ack_in[15]); router5.outack1(si_ack_in[16]); router5.rclk(r_clock); //need 64 code statement router router6("router6"); // hooking up signals to ports by name router6.in0(si_source[6]); router6.in1(si_output[6]); router6.in2(si_output[15]); router6.in3(si_output[22]); router6.in4(si_output[31]); router6.router_id(id6); //router2.router_id(2); router6.out0(si_sink[6]); router6.out1(si_output[17]); router6.out2(si_output[18]); router6.out3(si_output[19]); router6.out4(si_output[20]); router6.inack0(si_ack_sink[6]); router6.inack1(si_ack_in[6]); router6.inack2(si_ack_in[15]); router6.inack3(si_ack_in[22]); router6.inack4(si_ack_in[31]); router6.outack0(si_ack_src[6]); router6.outack1(si_ack_in[17]); router6.outack2(si_ack_in[18]); router6.outack3(si_ack_in[19]); router6.outack4(si_ack_in[20]); router6.rclk(r_clock); router router7("router7"); // hooking up signals to ports by name router7.in0(si_source[7]); router7.in1(si_output[8]); router7.in2(si_output[18]); router7.in3(si_output[35]); router7.in4(si_zero[11]); router7.router_id(id7); //router3.router_id(3); router7.out0(si_sink[7]); router7.out1(si_output[21]); router7.out4(si_output[22]); router7.out3(si_output[23]); router7.out2(si_zero[23]); router7.inack0(si_ack_sink[7]); router7.inack1(si_ack_in[8]); router7.inack2(si_ack_in[18]); router7.inack3(si_ack_in[35]); router7.inack4(si_ack_zero[11]); router7.outack0(si_ack_src[7]); router7.outack1(si_ack_in[21]); router7.outack4(si_ack_in[22]); router7.outack3(si_ack_in[23]); router7.outack2(si_ack_zero[23]); router7.rclk(r_clock); //------------------------------------------------------------------- router router8("router8"); // hooking up signals to ports by name router8.in0(si_source[8]); router8.in1(si_output[11]); router8.in2(si_output[27]); router8.in3(si_output[38]); router8.in4(si_zero[12]); router8.router_id(id8); //router0.router_id(0); router8.out0(si_sink[8]); router8.out2(si_output[24]); router8.out3(si_output[25]); router8.out1(si_output[26]); router8.out4(si_zero[24]); router8.inack0(si_ack_sink[8]); router8.inack1(si_ack_in[11]); router8.inack2(si_ack_in[27]); router8.inack3(si_ack_in[38]); router8.inack4(si_ack_zero[12]); router8.outack0(si_ack_src[8]); router8.outack2(si_ack_in[24]); router8.outack3(si_ack_in[25]); router8.outack1(si_ack_in[26]); router8.outack4(si_ack_zero[24]); router8.rclk(r_clock); router router9("router9"); // hooking up signals to ports by name router9.in0(si_source[9]); router9.in1(si_output[14]); router9.in2(si_output[24]); router9.in3(si_output[34]); router9.in4(si_output[41]); router9.router_id(id9); //router1.router_id(1); router9.out0(si_sink[9]); router9.out4(si_output[27]); router9.out3(si_output[28]); router9.out2(si_output[29]); router9.out1(si_output[30]); router9.inack0(si_ack_sink[9]); router9.inack1(si_ack_in[14]); router9.inack2(si_ack_in[24]); router9.inack3(si_ack_in[34]); router9.inack4(si_ack_in[41]); router9.outack0(si_ack_src[9]); router9.outack4(si_ack_in[27]); router9.outack3(si_ack_in[28]); router9.outack2(si_ack_in[29]); router9.outack1(si_ack_in[30]); router9.rclk(r_clock); //need 64 code statement router router10("router10"); // hooking up signals to ports by name router10.in0(si_source[10]); router10.in1(si_output[19]); router10.in2(si_output[29]); router10.in3(si_output[36]); router10.in4(si_output[43]); router10.router_id(id10); //router2.router_id(2); router10.out0(si_sink[10]); router10.out1(si_output[31]); router10.out2(si_output[32]); router10.out3(si_output[33]); router10.out4(si_output[34]); router10.inack0(si_ack_sink[10]); router10.inack1(si_ack_in[19]); router10.inack2(si_ack_in[29]); router10.inack3(si_ack_in[36]); router10.inack4(si_ack_in[43]); router10.outack0(si_ack_src[10]); router10.outack1(si_ack_in[31]); router10.outack2(si_ack_in[32]); router10.outack3(si_ack_in[33]); router10.outack4(si_ack_in[34]); router10.rclk(r_clock); router router11("router11"); // hooking up signals to ports by name router11.in0(si_source[11]); router11.in1(si_output[23]); router11.in2(si_output[32]); router11.in3(si_output[46]); router11.in4(si_zero[13]); router11.router_id(id11); //router3.router_id(3); router11.out0(si_sink[11]); router11.out1(si_output[35]); router11.out4(si_output[36]); router11.out3(si_output[37]); router11.out2(si_zero[25]); router11.inack0(si_ack_sink[11]); router11.inack1(si_ack_in[23]); router11.inack2(si_ack_in[32]); router11.inack3(si_ack_in[46]); router11.inack4(si_ack_zero[13]); router11.outack0(si_ack_src[11]); router11.outack1(si_ack_in[35]); router11.outack4(si_ack_in[36]); router11.outack3(si_ack_in[37]); router11.outack2(si_ack_zero[25]); router11.rclk(r_clock); //------------------------------------------------- router router12("router12"); // hooking up signals to ports by name router12.in0(si_source[12]); router12.in1(si_output[25]); router12.in2(si_output[40]); router12.in3(si_zero[14]); router12.in4(si_zero[15]); router12.router_id(id12); //router0.router_id(0); router12.out0(si_sink[12]); router12.out1(si_output[38]); router12.out2(si_output[39]); router12.out3(si_zero[26]); router12.out4(si_zero[27]); router12.inack0(si_ack_sink[12]); router12.inack1(si_ack_in[25]); router12.inack2(si_ack_in[40]); router12.inack3(si_ack_zero[14]); router12.inack4(si_ack_zero[15]); router12.outack0(si_ack_src[12]); router12.outack1(si_ack_in[38]); router12.outack2(si_ack_in[39]); router12.outack3(si_ack_zero[26]); router12.outack4(si_ack_zero[27]); router12.rclk(r_clock); router router13("router13"); // hooking up signals to ports by name router13.in0(si_source[13]); router13.in1(si_output[28]); router13.in2(si_output[39]); router13.in3(si_output[45]); router13.in4(si_zero[16]); router13.router_id(id13); //router1.router_id(1); router13.out0(si_sink[13]); router13.out4(si_output[40]); router13.out1(si_output[41]); router13.out2(si_output[42]); router13.out3(si_zero[28]); router13.inack0(si_ack_sink[13]); router13.inack1(si_ack_in[28]); router13.inack2(si_ack_in[39]); router13.inack3(si_ack_in[45]); router13.inack4(si_ack_zero[16]); router13.outack0(si_ack_src[13]); router13.outack4(si_ack_in[40]); router13.outack1(si_ack_in[41]); router13.outack2(si_ack_in[42]); router13.outack3(si_ack_zero[28]); router13.rclk(r_clock); //need 64 code statement router router14("router14"); // hooking up signals to ports by name router14.in0(si_source[14]); router14.in1(si_output[33]); router14.in2(si_output[42]); router14.in3(si_output[47]); router14.in4(si_zero[17]); router14.router_id(id14); //router2.router_id(2); router14.out0(si_sink[14]); router14.out1(si_output[43]); router14.out2(si_output[44]); router14.out4(si_output[45]); router14.out3(si_zero[29]); router14.inack0(si_ack_sink[14]); router14.inack1(si_ack_in[33]); router14.inack2(si_ack_in[42]); router14.inack3(si_ack_in[47]); router14.inack4(si_ack_zero[17]); router14.outack0(si_ack_src[14]); router14.outack1(si_ack_in[43]); router14.outack2(si_ack_in[44]); router14.outack4(si_ack_in[45]); router14.outack3(si_ack_zero[29]); router14.rclk(r_clock); router router15("router15"); // hooking up signals to ports by name router15.in0(si_source[15]); router15.in1(si_output[37]); router15.in2(si_output[44]); router15.in3(si_zero[18]); router15.in4(si_zero[19]); router15.router_id(id15); //router3.router_id(3); router15.out0(si_sink[15]); router15.out1(si_output[46]); router15.out4(si_output[47]); router15.out2(si_zero[30]); router15.out3(si_zero[31]); router15.inack0(si_ack_sink[15]); router15.inack1(si_ack_in[37]); router15.inack2(si_ack_in[44]); router15.inack3(si_ack_zero[18]); router15.inack4(si_ack_zero[19]); router15.outack0(si_ack_src[15]); router15.outack1(si_ack_in[46]); router15.outack4(si_ack_in[47]); router15.outack2(si_ack_zero[30]); router15.outack3(si_ack_zero[31]); router15.rclk(r_clock); sink sink0("sink0"); sink0(si_sink[0], si_ack_sink[0], siid0, d_clock, sioutput[0]); sink sink1("sink1"); sink1(si_sink[1], si_ack_sink[1], siid1, d_clock, sioutput[1]); sink sink2("sink2"); sink2(si_sink[2], si_ack_sink[2], siid2, d_clock, sioutput[2]); sink sink3("sink3"); sink3(si_sink[3], si_ack_sink[3], siid3, d_clock, sioutput[3]); sink sink4("sink4"); sink4(si_sink[4], si_ack_sink[4], siid4, d_clock, sioutput[4]); sink sink5("sink5"); sink5(si_sink[5], si_ack_sink[5], siid5, d_clock, sioutput[5]); sink sink6("sink6"); sink6(si_sink[6], si_ack_sink[6], siid6, d_clock, sioutput[6]); sink sink7("sink7"); sink7(si_sink[7], si_ack_sink[7], siid7, d_clock, sioutput[7]); sink sink8("sink8"); sink8(si_sink[8], si_ack_sink[8], siid8, d_clock, sioutput[8]); sink sink9("sink9"); sink9(si_sink[9], si_ack_sink[9], siid9, d_clock, sioutput[9]); sink sink10("sink10"); sink10(si_sink[10], si_ack_sink[10], siid10, d_clock, sioutput[10]); sink sink11("sink11"); sink11(si_sink[11], si_ack_sink[11], siid11, d_clock, sioutput[11]); sink sink12("sink12"); sink12(si_sink[12], si_ack_sink[12], siid12, d_clock, sioutput[12]); sink sink13("sink13"); sink13(si_sink[13], si_ack_sink[13], siid13, d_clock, sioutput[13]); sink sink14("sink14"); sink14(si_sink[14], si_ack_sink[14], siid14, d_clock, sioutput[14]); sink sink15("sink15"); sink15(si_sink[15], si_ack_sink[15], siid15, d_clock, sioutput[15]); //sc_start(0, SC_NS); // tracing: // trace file creation sc_trace_file *tf = sc_create_vcd_trace_file("graph"); // External Signals sc_trace(tf, s_clock, "s_clock"); sc_trace(tf, d_clock, "d_clock"); sc_trace(tf, r_clock, "r_clock"); sc_trace(tf, si_source[0], "si_source[0]"); sc_trace(tf, si_source[1], "si_source[1]"); sc_trace(tf, si_source[2], "si_source[2]"); sc_trace(tf, si_source[3], "si_source[3]"); sc_trace(tf, si_source[4], "si_source[4]"); sc_trace(tf, si_source[5], "si_source[5]"); sc_trace(tf, si_source[6], "si_source[6]"); sc_trace(tf, si_source[7], "si_source[7]"); sc_trace(tf, si_source[8], "si_source[8]"); sc_trace(tf, si_source[9], "si_source[9]"); sc_trace(tf, si_source[10], "si_source[10]"); sc_trace(tf, si_source[11], "si_source[11]"); sc_trace(tf, si_source[12], "si_source[12]"); sc_trace(tf, si_source[13], "si_source[13]"); sc_trace(tf, si_source[14], "si_source[14]"); sc_trace(tf, si_source[15], "si_source[15]"); sc_trace(tf, si_sink[0], "si_sink[0]"); sc_trace(tf, si_sink[1], "si_sink[1]"); sc_trace(tf, si_sink[2], "si_sink[2]"); sc_trace(tf, si_sink[3], "si_sink[3]"); sc_trace(tf, si_sink[4], "si_sink[4]"); sc_trace(tf, si_sink[5], "si_sink[5]"); sc_trace(tf, si_sink[6], "si_sink[6]"); sc_trace(tf, si_sink[7], "si_sink[7]"); sc_trace(tf, si_sink[8], "si_sink[8]"); sc_trace(tf, si_sink[9], "si_sink[9]"); sc_trace(tf, si_sink[10], "si_sink[10]"); sc_trace(tf, si_sink[11], "si_sink[11]"); sc_trace(tf, si_sink[12], "si_sink[12]"); sc_trace(tf, si_sink[13], "si_sink[13]"); sc_trace(tf, si_sink[14], "si_sink[14]"); sc_trace(tf, si_sink[15], "si_sink[15]"); cout<< "Enter source ID (0 to 15): "; cin >> i; cout<<"Enter Destination ID (0 to 15): "; cin >> j; scinput.write(j); id0.write(0); id1.write(1); id2.write(2); id3.write(3); id4.write(4); id5.write(5); id6.write(6); id7.write(7); id8.write(8); id9.write(9); id10.write(10); id11.write(11); id12.write(12); id13.write(13); id14.write(14); id15.write(15); check.write(i); scid0.write(0); scid1.write(1); scid2.write(2); scid3.write(3); scid4.write(4); scid5.write(5); scid6.write(6); scid7.write(7); scid8.write(8); scid9.write(9); scid10.write(10); scid11.write(11); scid12.write(12); scid13.write(13); scid14.write(14); scid15.write(15); siid0.write(0); siid1.write(1); siid2.write(2); siid3.write(3); siid4.write(4); siid5.write(5); siid6.write(6); siid7.write(7); siid8.write(8); siid9.write(9); siid10.write(10); siid11.write(11); siid12.write(12); siid13.write(13); siid14.write(14); siid15.write(15); cout << endl; cout << "-------------------------------------------------------------------------------" << endl; cout << endl << " 4X4 mesh NOC simulator containing 2 5x5 Wormhole router " << endl; cout << "-------------------------------------------------------------------------------" << endl; cout << "This is the simulation of a 4x4 Wormhole router. " << endl; cout << "We assume the router has 5 input/output ports, with 4 buffers per input port " << endl; cout << "and each flit has 21 bits width " << endl; cout << " Press \"Return\" key to start the simulation..." << endl << endl; getchar(); sc_start(10*125+124,SC_NS); // during [(10*125)+124] ns 10 packets will be sent and received sc_close_vcd_trace_file(tf); cout << endl << endl << "-------------------------------------------------------------------------------" << endl; cout << "End of switch operation..." << endl; if(i==0)cout << "Total number of packets sent: " << source0.pkt_snt<< endl; if(j==0)cout << "Total number of packets received: " << sink0.pkt_recv<< endl; if(i==1)cout << "Total number of packets sent: " << source1.pkt_snt<< endl; if(j==1)cout << "Total number of packets received: " << sink1.pkt_recv<< endl; if(i==2)cout << "Total number of packets sent: " << source2.pkt_snt<< endl; if(j==2)cout << "Total number of packets received: " << sink2.pkt_recv<< endl; if(i==3)cout << "Total number of packets sent: " << source3.pkt_snt<< endl; if(j==3)cout << "Total number of packets received: " << sink3.pkt_recv<< endl; if(i==4)cout << "Total number of packets sent: " << source4.pkt_snt<< endl; if(j==4)cout << "Total number of packets received: " << sink4.pkt_recv<< endl; if(i==5)cout << "Total number of packets sent: " << source5.pkt_snt<< endl; if(j==5)cout << "Total number of packets received: " << sink5.pkt_recv<< endl; if(i==6)cout << "Total number of packets sent: " << source6.pkt_snt<< endl; if(j==6)cout << "Total number of packets received: " << sink6.pkt_recv<< endl; if(i==7)cout << "Total number of packets sent: " << source7.pkt_snt<< endl; if(j==7)cout << "Total number of packets received: " << sink7.pkt_recv<< endl; if(i==8)cout << "Total number of packets sent: " << source8.pkt_snt<< endl; if(j==8)cout << "Total number of packets received: " << sink8.pkt_recv<< endl; if(i==9)cout << "Total number of packets sent: " << source9.pkt_snt<< endl; if(j==9)cout << "Total number of packets received: " << sink9.pkt_recv<< endl; if(i==10)cout << "Total number of packets sent: " << source10.pkt_snt<< endl; if(j==10)cout << "Total number of packets received: " << sink10.pkt_recv<< endl; if(i==11)cout << "Total number of packets sent: " << source11.pkt_snt<< endl; if(j==11)cout << "Total number of packets received: " << sink11.pkt_recv<< endl; if(i==12)cout << "Total number of packets sent: " << source12.pkt_snt<< endl; if(j==12)cout << "Total number of packets received: " << sink12.pkt_recv<< endl; if(i==13)cout << "Total number of packets sent: " << source13.pkt_snt<< endl; if(j==13)cout << "Total number of packets received: " << sink13.pkt_recv<< endl; if(i==14)cout << "Total number of packets sent: " << source14.pkt_snt<< endl; if(j==14)cout << "Total number of packets received: " << sink14.pkt_recv<< endl; if(i==15)cout << "Total number of packets sent: " << source15.pkt_snt<< endl; if(j==15)cout << "Total number of packets received: " << sink15.pkt_recv<< endl; cout << "-------------------------------------------------------------------------------" << endl; cout << " Press \"Return\" key to end the simulation..." << endl << endl; getchar(); return 0; }

#### arbiter.cpp

//arbiter.cpp #undef SC_INCLUDE_FX #include "packet.h" #include "arbiter.h" void arbiter :: func() { sc_uint<1> v_connected_input[5]; //set when input is connected to an output sc_uint<1> v_reserved_output[6]; //set when output is reserved by a input (one output more for simple coding) sc_uint<4> v_req[5]; sc_uint<5> v_free; // status of output in term of being free sc_uint<4> v_id; sc_uint<5> v_arbit; sc_uint<15> v_select; for(int i=0;i<5;i++){v_connected_input[i]=0;v_reserved_output[i]=0;v_req[i]=0;} v_free = 31; // '11111' v_arbit = 0; v_select = 0; // functionality while( true ) { wait(); grant0.write(0); // reset grant grant1.write(0); // reset grant grant2.write(0); // reset grant grant3.write(0); // reset grant grant4.write(0); // reset grant if (!free_out0.read()) {v_free = v_free ¦ 1 ; } // set the bit 0 showing the output 0 is free if (!free_out1.read()) {v_free = v_free ¦ 2 ; } if (!free_out2.read()) {v_free = v_free ¦ 4 ; } if (!free_out3.read()) {v_free = v_free ¦ 8 ; } if (!free_out4.read()) {v_free = v_free ¦ 16 ;} //for(int i=0;i<5;i++){cout<< "Request"<< i << req0.read()[i] <<endl;} v_id = arbiter_id.read(); //cout <<" ID "<< v_id <<endl; if (!req0.read()[4]) //if FIFO buffer is not empty { //if(!v_connected_input[0]) // if input is not connected i.e. it is header if(v_id[1] < req0.read()[1]) v_req[0]=3; // go to east else { if(v_id[1] > req0.read()[1])v_req[0]=5; //go to west else{ if(v_id[3] < req0.read()[3])v_req[0]=4; // go to south else{ if(v_id[3] > req0.read()[3])v_req[0]=2; //go to north else{ if(v_id[2] < req0.read()[2])v_req[0]=4; // go to south else{ if(v_id[2] > req0.read()[2])v_req[0]=2; //go to north else{ if(v_id[0] < req0.read()[0]) v_req[0]=3;// go to east else{ if(v_id[0] > req0.read()[0])v_req[0]=5; //go to west else v_req[0]=1; // that is the destination } } } } } } } switch (v_req[0]) { case 1: v_arbit=v_free & 1; break; case 2: v_arbit=v_free & 2; break; case 3: v_arbit=v_free & 4; break; case 4: v_arbit=v_free & 8; break; case 5: v_arbit=v_free & 16; break; default: break ; } if(!v_connected_input[0]) // if input is not connected // isnt this always 0 if its been intialized as 0. { if (v_reserved_output[v_req[0]])v_arbit=0; // if the requested output was reserved, go to next input } if(v_arbit!=0){ grant0.write(1); // set grant v_select.range(2,0) = v_req[0]; v_free = v_free & (~v_arbit); // inactive the related output v_connected_input[0]=1; // input 0 is connected v_reserved_output[v_req[0]]=1; // output is reserved if(req0.read()[5]){ v_connected_input[0]=0;v_reserved_output[v_req[0]]=0;} // if it is tail flit, reset connection and reservation } } if (!req1.read()[4]) //if buffer is not empty { //if(!v_connected_input[1]) // if input is not connected i.e. it is header if(v_id[1] < req1.read()[1]) v_req[1]=3; // go to east else { if(v_id[1] > req1.read()[1])v_req[1]=5; //go to west else { if(v_id[3] < req1.read()[3])v_req[1]=4; // go to south else { if(v_id[3] > req1.read()[3])v_req[1]=2; //go to north else{ if(v_id[2] < req1.read()[2])v_req[1]=4; // go to south else{ if(v_id[2] > req1.read()[2])v_req[1]=2; //go to north else{ if(v_id[0] < req1.read()[0]) v_req[1]=3; // go to east else{ if(v_id[0] > req1.read()[0])v_req[1]=5; //go to west else v_req[1]=1; // that is the destination } } } } } } } switch (v_req[1]) { case 1: v_arbit=v_free & 1; break; case 2: v_arbit=v_free & 2; break; case 3: v_arbit=v_free & 4; break; case 4: v_arbit=v_free & 8; break; case 5: v_arbit=v_free & 16; break; default: break ; } if(!v_connected_input[1]) // if input is not connected { if (v_reserved_output[v_req[1]])v_arbit=0; // if the requested output was reserved, go to next input } if(v_arbit!=0){ // if there is any free output grant1.write(1); // set grant v_select.range(5,3) = v_req[1]; v_free = v_free & (~v_arbit); // inactive the related outputs v_connected_input[1]=1; // input 1 is connected v_reserved_output[v_req[1]]=1; // output is reserved if(req1.read()[5]){v_connected_input[1]=0;v_reserved_output[v_req[1]]=0;} // if it is tail flit, reset connection and reservation } } if (!req2.read()[4]) //if buffer is not empty { //if(!v_connected_input[2]) // if input is not connected i.e. it is header if(v_id[1] < req2.read()[1]) v_req[2]=3; // go to east else { if(v_id[1] > req2.read()[1])v_req[2]=5; //go to west else { if(v_id[3] < req2.read()[3])v_req[2]=4; // go to south else { if(v_id[3] > req2.read()[3])v_req[2]=2; //go to north else{ if(v_id[2] < req2.read()[2])v_req[2]=4; // go to south else{ if(v_id[2] > req2.read()[2])v_req[2]=2; //go to north else{ if(v_id[0] < req2.read()[0]) v_req[2]=3; // go to east else{ if(v_id[1] > req2.read()[1])v_req[2]=5; //go to west else v_req[2]=1; // that is the destination } } } } } } } switch (v_req[2]) { case 1: v_arbit=v_free & 1; break; case 2: v_arbit=v_free & 2; break; case 3: v_arbit=v_free & 4; break; case 4: v_arbit=v_free & 8; break; case 5: v_arbit=v_free & 16; break; default: break ; } if(!v_connected_input[2]) // if input is not connected { if (v_reserved_output[v_req[2]])v_arbit=0; // if the requested output was reserved, go to next input } if(v_arbit!=0){ grant2.write(1); // set grant v_select.range(8,6) = v_req[2]; v_free = v_free & (~v_arbit); // inactive the related outputs v_connected_input[2]=1; // input 1 is connected v_reserved_output[v_req[2]]=1; // output is reserved //if(req2.read()[5]){v_connected_input[2]=0;v_reserved_output[v_req[2]]=0;} // if it is tail flit, reset connection and reservation } } if (!req3.read()[4]) //if buffer is not empty { //if(!v_connected_input[3]) // if input is not connected i.e. it is header if(v_id[1] < req3.read()[1]) v_req[3]=3; // go to east else { if(v_id[1] > req3.read()[1])v_req[3]=5; //go to west else { if(v_id[3] < req3.read()[3])v_req[3]=4; // go to south else { if(v_id[3] > req3.read()[3])v_req[3]=2; //go to north else{ if(v_id[2] < req3.read()[2])v_req[3]=4; // go to south else{ if(v_id[2] > req3.read()[2])v_req[3]=2; //go to north else{ if(v_id[0] < req3.read()[0]) v_req[3]=3; // go to east else{ if(v_id[0] > req3.read()[0])v_req[3]=5; //go to west else v_req[3]=1; // that is the destination } } } } } } } switch (v_req[3]) { case 1: v_arbit=v_free & 1; break; case 2: v_arbit=v_free & 2; break; case 3: v_arbit=v_free & 4; break; case 4: v_arbit=v_free & 8; break; case 5: v_arbit=v_free & 16; break; default: break ; } if(!v_connected_input[3]) // if input is not connected { if (v_reserved_output[v_req[3]])v_arbit=0; // if the requested output was reserved, go to next input } if(v_arbit!=0){ grant3.write(1); // set grant v_select.range(11,9) = v_req[3]; v_free = v_free & (~v_arbit); // inactive the related outputs v_connected_input[3]=1; // input 3 is connected v_reserved_output[v_req[3]]=1; // output is reserved if(req3.read()[5]){v_connected_input[3]=0;v_reserved_output[v_req[3]]=0;} // if it is tail flit, reset connection and reservation } } if (!req4.read()[4]) //if buffer is not empty { //if(!v_connected_input[4]) // if input is not connected i.e. it is header if(v_id[1] < req4.read()[1]) v_req[4]=3; // go to east else { if(v_id[1] > req4.read()[1])v_req[4]=5; //go to west else { if(v_id[3] < req4.read()[3])v_req[4]=4; // go to south else { if(v_id[3] > req4.read()[3])v_req[4]=2; //go to north else{ if(v_id[2] < req4.read()[2])v_req[4]=4; // go to south else{ if(v_id[2] > req4.read()[2])v_req[4]=2; //go to north else{ if(v_id[0] < req4.read()[0]) v_req[4]=3; // go to east else{ if(v_id[0] > req4.read()[0])v_req[4]=5; //go to west else v_req[4]=1; // that is the destination } } } } } } } switch (v_req[4]) { case 1: v_arbit=v_free & 1; break; case 2: v_arbit=v_free & 2; break; case 3: v_arbit=v_free & 4; break; case 4: v_arbit=v_free & 8; break; case 5: v_arbit=v_free & 16; break; default: break ; } if(!v_connected_input[4]) // if input is not connected { if (v_reserved_output[v_req[4]])v_arbit=0; // if the requested output was reserved, go to next input } if(v_arbit!=0){ grant4.write(1); // set grant v_select.range(14,12) = v_req[4]; v_free = v_free & (~v_arbit); // inactive the related outputs v_connected_input[4]=1; // input 4 is connected v_reserved_output[v_req[4]]=1; // output is reserved if(req4.read()[5]){v_connected_input[4]=0;v_reserved_output[v_req[4]]=0;} // if it is tail flit, reset connection and reservation } } aselect.write(v_select); } }

#### buf_fifo.cpp

// buf_fifo.cpp #include "buf_fifo.h" void fifo::packet_in(const packet& data_packet) { registers[reg_num++] = data_packet; empty = false; if (reg_num == 4) full = true; } packet fifo::packet_out() { reg_num--; packet temp; temp = registers[0]; if (reg_num == 0) empty = true; else { registers[0] = registers[1]; registers[1] = registers[2]; registers[2] = registers[3]; } full = false; return(temp); } void buf_fifo :: func() { fifo q0; // define a FIFO buffer q0.reg_num = 0; q0.full = false; q0.empty = true; packet b_temp; req.write((q0.registers[0].h_t,q0.empty, q0.registers[0].dest));// this outputs header or tail flit while empty and destination ID //cout << "header/tail flit" << q0.registers[0].h_t <<endl; // functionality while( true ) { wait(); /////received packet///// if (wr.event()) { q0.packet_in(wr.read()); ack.write(q0.full); req.write((q0.registers[0].h_t, q0.empty, q0.registers[0].dest)); //cout << "header/tail flit" << q0.registers[0].h_t <<endl; } ///// sent packets out///// if (bclk.event()) { // req.write((q0.empty, q0.registers[0].dest)); if(grant.read() == 1) { b_temp = q0.packet_out(); re.write(b_temp); ack.write(q0.full); req.write((q0.registers[0].h_t, q0.empty, q0.registers[0].dest)); //cout << "header/tail flit" << q0.registers[0].h_t <<endl; } } } }

#### router.cpp

// router.cpp #include "router.h" #define SIM_NUM 30 void router :: func() { int sim_count; // FILE *result; // initialization sim_count = 0; // result = fopen("result","w"); // functionality while( sim_count++ < SIM_NUM ) { wait(); if (in0.event()){pkt_sent++;}//cout << " source "<< in0.read().id << " router" << router_id.read()<< endl;} if (in1.event()){pkt_sent++;}//cout << " source "<< in1.read().id << " router" << router_id.read()<< endl;} if (in2.event()){pkt_sent++;}//cout << " source "<< in2.read().id << " router" << router_id.read()<< endl;} if (in3.event()){pkt_sent++;}//cout << " source "<< in3.read().id << " router" << router_id.read()<< endl;} if (in4.event()){pkt_sent++;}//cout << " source "<< in4.read().id << " router" << router_id.read()<< endl;} } // sc_stop(); }

#### crossbar.cpp

// crossbar.cpp #include "packet.h" #include "crossbar.h" void crossbar :: func() { packet v_cross0; packet v_cross1; packet v_cross2; packet v_cross3; packet v_cross4; sc_uint<15> v_config; // functionality while( true ) { wait(); v_config = config.read(); if (i0.event()) { v_cross0 = i0.read(); switch (v_config(2,0)) { case 1: o0.write(v_cross0); break; case 2: o1.write(v_cross0); break; case 3: o2.write(v_cross0); break; case 4: o3.write(v_cross0); break; case 5: o4.write(v_cross0); break; default: cout << "---------------------------------wrong destination " <<endl ;break ; } } if (i1.event()) { v_cross1 = i1.read(); switch (v_config(5,3)) { case 1: o0.write(v_cross1); break; case 2: o1.write(v_cross1); break; case 3: o2.write(v_cross1); break; case 4: o3.write(v_cross1); break; case 5: o4.write(v_cross1); break; default: cout << "------------------------------------wrong destination " <<endl; break ; } } if (i2.event()) { v_cross2 = i2.read(); switch (v_config(8,6)) { case 1: o0.write(v_cross2); break; case 2: o1.write(v_cross2); break; case 3: o2.write(v_cross2); break; case 4: o3.write(v_cross2); break; case 5: o4.write(v_cross2); break; default: cout << "---------------------------------wrong destination " <<endl; break ; } } if (i3.event()) { v_cross3 = i3.read(); switch (v_config(11,9)) { case 1: o0.write(v_cross3); break; case 2: o1.write(v_cross3); break; case 3: o2.write(v_cross3); break; case 4: o3.write(v_cross3); break; case 5: o4.write(v_cross3); break; default: cout << "----------------------------------wrong destination " <<endl; break ; } } if (i4.event()) { v_cross4 = i4.read(); switch (v_config(14,12)) { case 1: o0.write(v_cross4); break; case 2: o1.write(v_cross4); break; case 3: o2.write(v_cross4); break; case 4: o3.write(v_cross4); break; case 5: o4.write(v_cross4); break; default: cout << "-------------------------------------wrong destination" <<endl ;break ; } } } }

#### sink.cpp

// sink.cpp #include "sink.h" void sink::receive_data(){ packet v_packet; sc_time t_recv; if ( sclk.event() ) ack_out.write(false); if (packet_in.event() ) { pkt_recv++ ; ack_out.write(true); v_packet= packet_in.read(); t_recv = sc_time_stamp(); cout << "\nt: " << sc_time_stamp() << " ¦¦ New Pkt: " << (int)v_packet.data<< " is received from source: " << (int)v_packet.id << " by sink: " << (int)sink_id.read() << endl << "____________________________________________________________________________________________"; } }

#### source.cpp

// source.cpp #include "source.h" void source:: func() { packet v_packet_out; v_packet_out.data=1000; // e.g. v_packet_out.pkt_clk = '0'; // an imaginary clock for packets sc_time t_sent; while(true) { wait(); if(!ach_in.read()) { if(ch_k.read() == source_id.read()) { v_packet_out.data = v_packet_out.data + 1 ; // made a desired data v_packet_out.id = source_id.read(); v_packet_out.dest= d_est.read(); // assign destination if(v_packet_out.id == v_packet_out.dest) goto exclode; // prevent from reciving flits by itself v_packet_out.pkt_clk= ~v_packet_out.pkt_clk ; // add an imaginary clock to each flit v_packet_out.h_t=false; pkt_snt++; if((pkt_snt%5)==0)v_packet_out.h_t=true; // make tail flit (the packet size is 5) packet_out.write(v_packet_out); t_sent = sc_time_stamp(); cout << "\nt: " << sc_time_stamp() << " ¦¦ New Pkt: " << v_packet_out.data << " is sent by source: " << source_id.read() << " to Destination: "<< v_packet_out.dest <<endl; exclode:; } } } }