Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE838 |
| Course Title | SOC Design |
| Semester/Year | W2026 |
| Instructor | Gul N. Khan |

| Lab/Tutorial Report NO. | Lab #2a Report |
| --- | --- |

| Report Title | Lab #2a: COE838: Systems-on-Chip Design LAB-2a |
| --- | --- |

| Section No. | 03 |
| --- | --- |
| Submission Date | February 2, 2025 |
| Due Date | February 3, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Abstract/Introduction

This lab focused on the creation of an 8x8 array multiplier accelerator using SystemC for the use of System-On-Chip applications. Using a Carry Save Adder (CSA) as well as a Carry Propagate Adder (CPA), the multiplier was constructed with a provided 4x4 multiplier architecture. By furthering the design to an 8x8 array, the signal connections and computation will be in order. A testbench validates the functionality of the multiplier with testing scenarios (average and corner cases). The resulting waveforms were analyzed to prove the accuracy of the multiplier.

Modern SoC architectures use hardware accelerators such as multipliers to improve upon latency and efficiency. Array multiplication is easy to implement, and gaining a fundamental understanding in the lab will present how digital hardware components can be represented/verified at a high level while maintaining cycle-accurate behavior. The importance of modular hardware design and verification in SoC development will be proven with a successful implementation of this accelerator.

## Methodology

The understanding of CSA and CPA units are extremely important when creating a multiplier in SystemC.

The key parts of the multiplier are:

- Full adder

With inputs A, B and Carry-in, C, and outputs Sum (So), and Carry-out (Co)

- CSA

Performs operation on A and B. Feeds/result along with Sum-in and Carry-in to a full adder

- CPA

With the full adder, it propagates carries in the final row

- 8x8 multiplier

CSA are in matrix form

CPA are in the final row (MSB generation)

With proper sequencing, there will be respect dependency between rows and columns

The testbench was created in the sc_main.cpp. It applies various inputs like zero values, max values, mid-range values, and power-of-twos.

The arrangement of the CSA blocks are in a two-dimensional matrix where each block will generate partial products using the AND logic with accumulated intermediate sums and carry values. The results are then passed diagonally and vertically across the array, after the dependency order needed from the architecture.

The last row of the matrix is made up of CPA blocks in order to propagate the remaining carry values, and to produce the most significant bits of the result. It was of utmost importance to make sure that the signal had proper sequencing to avoid improper propagation of carries from rows and columns.

## Results

Figure 1: GTKWave Simulation Results of the 8×8 SystemC Array Multiplier

Figure 2: Waveform for Test Case 1 (A = 3, B = 2) (0-20ns)

Figure 3: Waveform for Test Case 2 (A = 0, B = 44) (20-40ns)

Figure 4: Waveform for Test Case 3 (A = 9, B = 9) (40-60ns)

Figure 1 shows the GTKWave output of the 8x8 SystemC array multiplier. From the waveform, CSA outputs A_input_whole_module[7:0] and B_input_whole_module[7:0] both update accordingly. CPA outputs (LSB_output_cpa[7:0] and MSB_output_cpa[7:0]) show similar results in terms of correctness. Product[15:0], the final signal, shows the correct multiplication results for each pair of inputs.

## Challenges, Outstanding Work, and Future Improvements

Challenges within this project included making sure the correct order of computation occurred between the rows and columns, or debugging signal mismatches during early versions of the CSA block. Additionally, there was a large amount of internal signals to connect the CSA and CPA blocks which made debugging the program complicated.

The multiplier can be improved by optimizing the design for fast performance so there will be fewer delays when running the program. Pipelining or parallel processing can be implemented to increase the program speed. One can also separate the CPA and CSA blocks into independent functions. This way they are also reusable for various projects.

## Discussion/Conclusion

An 8x8 array multiplier was successfully designed and implemented. CSA and CPA blocks allowed efficient partial product accumulation and carry propagation, while simulation results confirmed that the project was fully functional. Hardware modeling in SystemC, hierarchical module design, and debugging complex digital systems were reinforced during this lab project. Experience in creating multipliers using SystemC can be applied to fundamental SoC accelerator components and an amazing understanding for many hardware design tasks.

## References

[1] Toronto Metropolitan University, Department of Electrical, Computer, and Biomedical Engineering, COE838: Systems-on-Chip Design – Lab 2a: SystemC-Based SoC Accelerator Design: The Multiplier, Toronto, ON, Canada, 2025.

## Appendix

### Arraymult.cpp

#include "CPA.h" #include "CSA.h" #define size 8 //CSA sc_uint<size> sum; sc_uint<size> carryout; sc_uint<size> ain; sc_uint<size> bin; sc_uint<size> A_combined; sc_uint<size> sin_input; sc_uint<size> ainandbin; sc_uint<size> lsb_val; sc_uint<size> carryin; sc_uint<size> sum_actual; sc_uint<size> carr_actual; sc_uint<size> sum_simplier; sc_uint<size> carr_simplier; int i = 0; int j = 0; int y = 0; int sum_holder[size][size]; int carryout_holder[size][size]; // CPA sc_uint<size> sum_cpa; sc_uint<size> carryout_cpa; sc_uint<size> sum_holder_cpa; sc_uint<size> carryout_holder_cpa; sc_uint<size> sum_csa; sc_uint<size> carryout_csa; sc_uint<size> ain_cpa; sc_uint<size> bin_cpa; sc_uint<size> carryin_cpa; sc_uint<size> least_significant; sc_uint<size * 2> complete_output; sc_uint<size> msb_val; int x, w = 0; int z = 0; // a xor b // b and c // (a xor b) xor cin => sum -> main eq // (a and c) OR (a and b) => cout -> main eq void CPA ::CPA_method() { cout << "\n===================== Array Multiplier Operation =====================\n"; //**************************************** CSA Method ain = A_input.read(); bin = B_input.read(); A_combined = ain & bin; for (i = 0; i < size; i++) { // row number for (j = 0; j < size; j++) { // column number if (i == 0) { // if first row , C = 0 and S = 0 sin_input = 0; carryin = 0; A_combined = ain[j] & bin[i]; sum = (((A_combined) ^ (sin_input)) ^ (carryin)); //((axorb)) xor cin carryout = ((A_combined)&sin_input) ¦ (sin_input & carryin) ¦ (A_combined & carryin); if (j == 0) { lsb_val[i] = sum[j]; // every row that last column sum will be stored in the lsb register } sum_holder[i][j] = sum; // holder current column number (i.e first column A3) carryout_holder[i][j] = carryout; // holder current column number (i.e first column A3) } else if (j == (size - 1) && (i != 0)) { // S = 0 A_combined = ain[j] & bin[i]; carryin = carryout_holder[i - 1][j]; // will have to make equal carryout value in same column but previous row sin_input = 0; sum = (((A_combined) ^ (sin_input)) ^ (carryin)); //((axorb)) xor cin carryout = ((A_combined)&sin_input) ¦ (sin_input & carryin) ¦ (A_combined & carryin); sum_holder[i][j] = sum; // holder current column number (i.e first column A3) carryout_holder[i][j] = carryout; // holder current column number (i.e first column A3) } else { A_combined = ain[j] & bin[i]; carryin = carryout_holder[i - 1][j]; // will have to make equal carryout value in same column but previous row sin_input = sum_holder[i - 1][j + 1]; // will have to make sin input the previous row & future columb sum = (((A_combined) ^ (sin_input)) ^ (carryin)); //((axorb)) xor cin carryout = ((A_combined)&sin_input) ¦ (sin_input & carryin) ¦ (A_combined & carryin); sum_holder[i][j] = sum; // holder current column number (i.e first column A3) carryout_holder[i][j] = carryout; // holder current column number (i.e first column A3) if (j == 0) { lsb_val[i] = sum[j]; // every row that last column sum will be stored in the lsb register } } cout << "CSA["<<i<<","<<j<<"] : "; cout << "A bit = " << ain[j] << ", B bit = " << bin[j] << " ¦ "; cout << "Carryin = "<< carryin << " ¦ "; cout << "Sumout = "<< sum_holder[i][j] << " ¦ "; cout << "Carryout = "<< carryout_holder[i][j] << " ¦ "; cout << endl; } } for (y = 0; y < size; y++) { // column sum_simplier[y] = sum_holder[size - 1][y]; carr_simplier[y] = carryout_holder[size - 1][y]; } lsb_out.write(lsb_val); carry_out_csa.write(carr_simplier); sum_out_csa.write(sum_simplier); //**************************************** CSA Method sum_csa = sum_simplier; carryout_csa = carr_simplier; least_significant = lsb_val; // sum_csa = A_input.read(); //in sc_main sum_csa will be inputted into CPA A_input // carryout_csa = B_input.read(); //in sc_main sum_csa will be inputted into CPA B_input // least_significant = least_sig.read(); for (x = 0; x < size; x++) { // x is column # if (x == 0) { // if A0 carryin_cpa = 0; ain_cpa = sum_csa[x + 1]; bin_cpa = carryout_csa[x]; sum_cpa = ((ain_cpa ^ bin_cpa) ^ (carryin_cpa)); carryout_cpa = ((ain_cpa & bin_cpa) ¦ (carryin_cpa & bin_cpa) ¦ (ain_cpa & carryin_cpa)); sum_holder_cpa[x] = sum_cpa; msb_val[x] = sum_cpa; carryout_holder_cpa[x] = carryout_cpa; } else if (x == (size - 1)) { // if x = A3 carryin_cpa = 0; bin_cpa = carryout_holder_cpa[x - 1]; ain_cpa = carryout_csa[x]; sum_cpa = ((ain_cpa ^ bin_cpa) ^ (carryin_cpa)); carryout_cpa = ((ain_cpa & bin_cpa) ¦ (carryin_cpa & bin_cpa) ¦ (ain_cpa & carryin_cpa)); sum_holder_cpa[x] = sum_cpa; msb_val[x] = sum_cpa; carryout_holder_cpa[x] = carryout_cpa; } else { carryin_cpa = carryout_holder_cpa[x - 1]; // making C in equal to prev value C out bin_cpa = carryout_csa[x]; ain_cpa = sum_csa[x + 1]; sum_cpa = ((ain_cpa ^ bin_cpa) ^ (carryin_cpa)); carryout_cpa = ((ain_cpa & bin_cpa) ¦ (carryin_cpa & bin_cpa) ¦ (ain_cpa & carryin_cpa)); sum_holder_cpa[x] = sum_cpa; msb_val[x] = sum_cpa; carryout_holder_cpa[x] = carryout_cpa; } cout << "CPA " << x << " sumout = " << sum_holder_cpa[x] << endl; } msb_out.write(msb_val); for (z = 0; z < (size * 2); z++) { if (z < (size)) { complete_output[z] = least_significant[z]; } else { complete_output[z] = msb_val[z - size]; } } complete.write(complete_output); cout << "Final output: "<< complete_output.to_int() << endl; }

### CPA.h

#ifndef CPA_H #define CPA_H #include <systemc.h> #include "CSA.h" #define size 8 SC_MODULE(CPA) { sc_in<sc_uint<size>> B_input; sc_in<sc_uint<size>> A_input; sc_in<sc_uint<size>> carry_in; sc_in<sc_uint<size>> least_sig; sc_out<sc_uint<size>> carry_out; sc_out<sc_uint<size>> sum_out; sc_out<sc_uint<size>> lsb_out; sc_out<sc_uint<size>> sum_out_csa; sc_out<sc_uint<size>> carry_out_csa; sc_out<sc_uint<size>> msb_out; sc_out<sc_uint<size * 2>> complete; void CPA_method(); SC_CTOR(CPA) { SC_METHOD(CPA_method); dont_initialize(); sensitive << A_input << B_input << carry_in; // edge sensitive (positive) } }; #endif

### CSA.h

#ifndef CSA_H #define CSA_H #include <systemc.h> #define size 8 SC_MODULE(CSA) { sc_in<sc_uint<size>> B_input; sc_in<sc_uint<size>> A_input; sc_in<sc_uint<size>> S_input; sc_in<sc_uint<size>> carry_in; sc_out<sc_uint<size>> carry_out; sc_out<sc_uint<size>> sum_out; sc_out<sc_uint<size>> lsb_out; void CSA_method(); SC_CTOR(CSA) { SC_METHOD(CSA_method); dont_initialize(); sensitive << A_input << B_input << S_input << carry_in; // edge sensitive (positive) } }; #endif

### Sc_main.cpp

// Main cpp #include <systemc.h> #include "CSA.h" #include "CPA.h" #define size 8 int sc_main(int argc, char *argv[]) { sc_trace_file *tf; // Create VCD file for tracing sc_signal<sc_uint<size>> sum_cpa; sc_signal<sc_uint<size>> carryout_cpa; sc_signal<sc_uint<size>> ain_cpa; sc_signal<sc_uint<size>> bin_cpa; sc_signal<sc_uint<size>> carryin_cpa; sc_signal<sc_uint<size>> lsb_out_cpa; sc_signal<sc_uint<size>> sum_out_csa; sc_signal<sc_uint<size>> carry_out_csa; sc_signal<sc_uint<size>> msb_out_cpa; sc_signal<sc_uint<size>> lsb_in_cpa; sc_signal<sc_uint<size * 2>> product; sc_signal<sc_uint<size>> sum_csa; sc_signal<sc_uint<size>> carryout_csa; sc_signal<sc_uint<size>> ain_csa; sc_signal<sc_uint<size>> bin_csa; sc_signal<sc_uint<size>> sin_csa; sc_signal<sc_uint<size>> carryin_csa; sc_signal<sc_uint<size>> lsb_out_csa; sc_clock clk("clk", 10, SC_NS, 0.5); // Create a clock signal CPA arraymult_DUT("CPA"); // Create Device Under Test (DUT) arraymult_DUT.B_input(bin_cpa); arraymult_DUT.A_input(ain_cpa); arraymult_DUT.carry_in(carryin_cpa); arraymult_DUT.carry_out(carryout_cpa); arraymult_DUT.sum_out(sum_cpa); arraymult_DUT.lsb_out(lsb_out_cpa); arraymult_DUT.sum_out_csa(sum_out_csa); arraymult_DUT.carry_out_csa(carry_out_csa); arraymult_DUT.msb_out(msb_out_cpa); arraymult_DUT.complete(product); arraymult_DUT.least_sig(lsb_in_cpa); // Create wave file and trace the signals executing tf = sc_create_vcd_trace_file("trace_file"); tf->set_time_unit(1, SC_NS); sc_trace(tf, msb_out_cpa, "MSB_output_cpa"); sc_trace(tf, product, "Product"); sc_trace(tf, sum_out_csa, "Sum_CSA_output"); sc_trace(tf, carry_out_csa, "Carry_CSA_output"); sc_trace(tf, bin_cpa, "B_input_whole_module"); sc_trace(tf, ain_cpa, "A_input_whole_module"); sc_trace(tf, lsb_out_cpa, "LSB_output_cpa"); cout << "\nExecuting Array Multiplier DUT... check .vcd produced" << endl; // Test normal product: 3 * 2 = 6 ain_cpa.write(3); bin_cpa.write(2); sc_start(20, SC_NS); // Test multiplying with 0 ain_cpa.write(0); bin_cpa.write(44); sc_start(20, SC_NS); // Test multiplying with same number and double digit result ain_cpa.write(9); bin_cpa.write(9); sc_start(20, SC_NS); // Test large number (max) multiplication ain_cpa.write(255); bin_cpa.write(255); sc_start(20, SC_NS); sc_close_vcd_trace_file(tf); return 0; }