Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE838 |
| Course Title | SOC Design |
| Semester/Year | W2026 |
| Instructor | Gul N. Khan |

| Lab/Tutorial Report NO. | Lab #3 Report |
| --- | --- |

| Report Title | Lab #2a: COE838: System-on-Chip Design Lab 2b |
| --- | --- |

| Section No. | 03 |
| --- | --- |
| Submission Date | March 2nd, 2025 |
| Due Date | March 3rd, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Abstract/Introduction

This lab will implement a SystemC-based JPEG encoder and decoder using a hardware/software co-design method. The created code will identify (computationally) intensive portions of the JPEG algorithm and implement them as dedicated hardware modules, while simpler operations are executed in software. By using both hardware and software to create this project, advantages of both mediums can be taken advantage of to create the most efficient result.

JPEG compression is extremely important in the world of engineering due to the ability to reduce image size. This lab uses a grayscale bitmap image, and compresses/decompresses it using a simplified JPEG pipeline consisting of Discrete Cosine Transform (DCT), quantization, zigzag ordering, and their inverse operations. Huffman encoding and decoding methods are not implemented to not let the project become overly complex.

## Methodology

A CPU (SC_MAIN) will be the main architecture to execute both the required hardware and software components. The JPEG compression/decompression will be done in two phases.

The first will be Compression Flow:

- Bitmap header extraction and transfer

- Image preprocessing:

  - Image divided into 8×8 blocks

  - Pixel values shifted from unsigned [0, 255] to signed [-128, 127]

- DCT (Hardware Module):

  - Each 8×8 block is passed to the DCT hardware accelerator

- Quantization (Software):

  - DCT coefficients divided by the standard JPEG quantization table

- Zigzag Ordering (Software):

  - Quantized coefficients reordered into a 1D array

- Compressed data written to output file [1]

The second will be Decompression Flow:

- Compressed data read from file

- Unzigzag (Software)

- Unquantization (Software)

- Inverse DCT (Hardware Module)

- Pixel values shifted back to unsigned range

- Bitmap reconstructed and written to output file [1]

“Functions.cpp” was used for all software operations. The given DCT and inverse DCT modules were used to compress and decompress the JPEG images.

## Results

Figure 1: Original Input Image

Figure 2: Compressed Output Image

Figure 3: Decompressed Output Image

The JPEG image is successfully compressed and decompressed. The compression was visually similar to the original and the decompression resulted in an image that was almost identical to the original; despite some quality degradation due to quantization.

The system correctly handled image headers and preserved image dimensions. The simulation confirmed proper synchronization with hardware and software components. No runtime errors were observed during execution.

## Challenges, Outstanding Work, and Future Improvements

A challenge encountered was understanding the correct order of JPEG processing stages, as incorrect sequencing can result in corrupted output images. Additionally using the DCT and inverse DCT properly is very important since they require careful signal binding, data transfer, or complex operations. Software with many components require attentive construction since one small mistake can create unexpected results.

To improve the program, Huffman encoding should be included so that true JPEG compression can be done. One can also extend support to color images (RGB) instead of grayscale only, or optimize hardware modules for pipelining and improved performance.

## Discussion/Conclusion
This lab created a proper JPEG compression and decompression demonstration. The correct result proves the effectiveness of a hardware/software co-design methodology using SystemC. The DCT and IDCT to hardware modules were offloaded, and system performance improved while maintaining flexibility through software execution of simpler tasks. 

The JPEG encoder and decoder properly followed the proper sequencing for standard compression and decompression, proving the design approach. This lab provides practical experience in image processing with SoC design, along with performance-driven system partitioning.

## References

[1] Toronto Metropolitan University, “COE838: System-on-Chip Design – Lab 2b Manual,” Dept. of Electrical, Computer, and Biomedical Engineering, TMU, Toronto, ON, Canada, 2025.

## Appendix

#### sc_main.cpp

#include "systemc.h" #include "functions.h" #include "fdct.h" #include "idct.h" // #include inverse module #define NS *1e-9 // use this constant to make sure that the clock signal is in nanoseconds int sc_main(int argc, char *argv[]) { char choice; sc_signal<FILE *> sc_input; // input file pointer signal sc_signal<FILE *> sc_output; // output file pointer signal sc_signal<double> dct_data[8][8]; // signal to the dc transformed data sc_signal<double> cosine_tbl[8][8]; // signal for the cosine table values sc_signal<bool> clk1, clk2; // clock signal for FDCT (also need a 2nd clock for IDCT) FILE *input, *output; // input and output file pointers double cosine[8][8]; // the cosine table double data[8][8]; // the data read from the signals, which will be zigzagged // Decompression signals and variables if (argc == 4) { if (!(input = fopen(argv[1], "rb"))) // error occurred while trying to open file printf("\nSystemC JPEG LAB:\ncannot open file '%s'\n", argv[1]), exit(1); if (!(output = fopen(argv[2], "wb"))) // rror occurred while trying to create file printf("\nSystemC JPEG LAB:\ncannot create file '%s'\n", argv[2]), exit(1); // copy the input and output file pointer onto the respective ports sc_input.write(input); sc_output.write(output); choice = *argv[3]; } else fprintf(stderr, "\nSystemC JPEG LAB: insufficient command line arguments\n" "usage: ./sc_jpeg.x [input file] [output file] [(C)ompress or (D)ecompress]\n"), exit(1); // write the header, read from the input file write_read_header(input, output); // make the cosine table make_cosine_tbl(cosine); // have to manually bind each value in the array, error when directly writing the whole array for (int i = 0; i < 8; ++i) { for (int j = 0; j < 8; ++j) { cosine_tbl[i][j].write(cosine[i][j]); } } // call the forward discrete transform module // and bind the ports -FDCT fdct fdct_module("fdct"); for (int i = 0; i < 8; ++i) { for (int j = 0; j < 8; ++j) { fdct_module.out64[i][j](dct_data[i][j]); } } for (int i = 0; i < 8; ++i) { for (int j = 0; j < 8; ++j) { fdct_module.fcosine[i][j](cosine_tbl[i][j]); } } fdct_module.sc_input(sc_input); fdct_module.clk(clk1); // copy the cosine table and the quantization table onto the corresponding signals to send to DCT module // binds ports - idct idct idct_module("idct"); for (int i = 0; i < 8; ++i) { for (int j = 0; j < 8; ++j) { idct_module.in64[i][j](dct_data[i][j]); } } for (int i = 0; i < 8; ++i) { for (int j = 0; j < 8; ++j) { idct_module.fcosine[i][j](cosine_tbl[i][j]); } } idct_module.sc_output(sc_output); idct_module.clk(clk2); // because compression and decompression are two differnt processes, we must use // two different clocks, to make sure that when we want to compress, we only compress // and dont decompress by mistake sc_start(SC_ZERO_TIME); // initialize the clock if ((choice == 'c') ¦¦ (choice == 'C')) { // for compression while (!(feof(input))) { // cycle the clock for as long as there is something to be read from the input file // create the FDCT clock signal clk1.write(1); // convert the clock to high sc_start(10, SC_NS); // cycle the high for 10 nanoseconds clk1.write(0); // start the clock as low sc_start(10, SC_NS); // cycle the low for 10 nanoseconds // read back signals from module for (int i = 0; i < 8; ++i) { for (int j = 0; j < 8; ++j) { data[i][j] = fdct_module.out64[i][j].read(); } } // zigzag and quantize the outputted data - will write out to file (see functions.h) zigzag_quant(data, output); } } else if ((choice == 'd') ¦¦ (choice == 'D')) { // for decompression while (!(feof(input))) { // unzigzag and inverse quatize input file and result will be placed in data unzigzag_iquant(data, input); // write unzigzag data to ports for (int i = 0; i < 8; ++i) { for (int j = 0; j < 8; ++j) { dct_data[i][j].write(data[i][j]); } } clk2.write(1); // convert the clock to high sc_start(10, SC_NS); // cycle the high for 10 nanoseconds clk2.write(0); // start the clock as low sc_start(10, SC_NS); // cycle the low for 10 nanoseconds // read idct data from ports & write to output file } } fclose(sc_input.read()); fclose(sc_output.read()); return 0; }

#### fdct.cpp

#include "fdct.h" void fdct :: calculate_dct( void ) { unsigned char u, v, x, y; double temp; // do forward discrete cosine transform for (u = 0; u < 8; u++) for (v = 0; v < 8; v++) { temp = 0.0; for (x = 0; x < 8; x++) for (y = 0; y < 8; y++) temp += input_data[x][y] * fcosine[x][u].read() * fcosine[y][v].read(); if ((u == 0) && (v == 0)) temp /= 8.0; else if (((u == 0) && (v != 0)) ¦¦ ((u != 0) && (v == 0))) temp /= (4.0*sqrt(2.0)); else temp /= 4.0; out64[u][v].write(temp); } printf("."); } void fdct :: read_data( void ) { // read the 8x8 block as a continuous 64 values and store them in // input_data as an 8x8 block fread(input_data, 1, 64, sc_input.read()); // shift the unsigned integers from range [0, 2^8 - 1] to signed integers // of range [2^(8-1), 2^(8-1) - 1] for (unsigned char uv = 0; uv < 64; uv++) input_data[uv/8][uv%8] -= (char) (pow(2,8-1)); }

fdct.h

#include "systemc.h" #include <math.h> #ifndef FDCT_H #define FDCT_H struct fdct : sc_module { sc_out<double> out64[8][8]; // the dc transformed 8x8 block sc_in<double> fcosine[8][8]; // cosine table input sc_in<FILE *> sc_input; // input file pointer port sc_in<bool> clk; // clock signal char input_data[8][8]; // the data read from the input file void read_data( void ); // read the 8x8 block void calculate_dct( void ); // perform dc transform // define fdct as a constructor SC_CTOR( fdct ) { // make read_data method sensitive to the positive clock edge, and // the calculate_dct method sensitive to the negative clock edge // this way, the entire read and performing dct takes only one clock cycle // as apposed to two SC_METHOD( read_data ); // define read_data as a method dont_initialize(); sensitive << clk.pos(); SC_METHOD( calculate_dct ); // define calculate_dct as a method dont_initialize(); sensitive << clk.neg(); } }; #endif

#### idct.cpp

#include "idct.h" void idct::calculate_idct() { unsigned char u, v, x, y; double temp; // Do inverse discrete cosine transform for (x = 0; x < 8; x++) { for (y = 0; y < 8; y++) { temp = 0.0; for (u = 0; u < 8; u++) { for (v = 0; v < 8; v++) { double cu = (u == 0) ? (1 / sqrt(2)) : 1; double cv = (v == 0) ? (1 / sqrt(2)) : 1; temp += cu * cv * in64[u][v].read() * fcosine[x][u].read() * fcosine[y][v].read(); } } output_data[x][y] = (char)round(temp / 4.0); } } // Write output_data to out64 // for (x = 0; x < 8; x++) { // for (y = 0; y < 8; y++) { // out64[x][y].write(output_data[x][y]); // } // } printf("."); } void idct::write_data() { FILE* output_file = sc_output.read(); // Write the output data to the output file fwrite(output_data, 1, 64, output_file); }

#### idct.h

#include "systemc.h" #include <math.h> #ifndef IDCT_H #define IDCT_H struct idct : sc_module { sc_in<double> in64[8][8]; // the input 8x8 block sc_out<FILE*> sc_output; // output file pointer port sc_in<double> fcosine[8][8]; // cosine table input sc_in<bool> clk; // clock signal char output_data[8][8]; // the data to be written to the output file void calculate_idct(); // perform inverse discrete cosine transform void write_data(); // write the output 8x8 block to the output file // Define idct as a constructor SC_CTOR(idct) { // Make calculate_idct method sensitive to the positive clock edge SC_METHOD(calculate_idct); dont_initialize(); sensitive << clk.pos(); // Make write_data method sensitive to the positive clock edge SC_METHOD(write_data); dont_initialize(); sensitive << clk.neg(); } }; #endif

#### functions.cpp

#include "functions.h" #define rnd(x) (((x)>=0)?((signed char)((signed char)((x)+1.5)-1)):((signed char)((signed char)((x)-1.5)+1))) #define rnd2(x) (((x)>=0)?((short int)((short int)((x)+1.5)-1)):((short int)((short int)((x)-1.5)+1))) #define PI 3.1415926535897932384626433832795 // the value of PI // the end of block marker, something which is highly unlikely to be found in a dct block signed char MARKER = 127; // quantization table unsigned char quant[8][8] = {{16,11,10,16,24,40,51,61}, {12,12,14,19,26,58,60,55}, {14,13,16,24,40,57,69,56}, {14,17,22,29,51,87,80,62}, {18,22,37,56,68,109,103,77}, {24,35,55,64,81,104,113,92}, {49,64,78,87,103,121,120,101}, {72,92,95,98,112,100,103,99}}; // zigzag table unsigned char zigzag_tbl[64] = { 0,1,5,6,14,15,27,28, 2,4,7,13,16,26,29,42, 3,8,12,17,25,30,41,43, 9,11,18,24,31,40,44,53, 10,19,23,32,39,45,52,54, 20,22,33,38,46,51,55,60, 21,34,37,47,50,56,59,61, 35,36,48,49,57,58,62,63 }; void write_read_header(FILE *in, FILE *out) { unsigned char temp[60]; // temporary array of 60 characters, which is enough // for the bitmap header, which is 54 bytes printf("\nInput Header read and written to the output file"); fread(temp, 1, 54, in); // read 54 bytes from the input file and store them in temp fwrite(temp, 1, 54, out); // write the 54 bytes to the output file printf("......Done\n"); printf("Image is a %d bit Image. Press Enter to Continue\n>", temp[28]); getchar(); } void make_cosine_tbl(double cosine[8][8]) { printf("Creating the cosine table for use in FDCT and IDCT"); // calculate the cosine table as defined in the formula for (unsigned char i = 0; i < 8; i++) for (unsigned char j = 0; j < 8; j++) cosine[i][j] = cos((((2*i)+1)*j*PI)/16); printf("......Done\n"); } void zigzag_quant(double data[8][8], FILE *output) { signed char to_write[8][8], final_write[8][8]; // this is the rounded values, to be written to the file char last_non_zero_value = 0; // this is the index to the last non-zero value in a block //quantization calculation with rounding (rnd) to nearest integer for (unsigned char i = 0; i < 8; i++) { for(unsigned char j = 0; j < 8; j++){ to_write[i][j] = rnd(data[i][j] / quant[i][j]); } } // zigzag the data array and copy it back to final_write array //find out the index to the last non-zero value in the 8x8 block for (unsigned char i = 0; i < 64; i++) { final_write[zigzag_tbl[i]/8][zigzag_tbl[i]%8] = to_write[i/8][i%8]; if (final_write[i/8][i%8] != 0) last_non_zero_value = i; } // write all the values in the block up to and including the last non-zero value for (unsigned char i = 0; i <= last_non_zero_value; i++) fwrite(&final_write[i/8][i%8], sizeof(signed char), 1, output); // write the end of block marker fwrite(&MARKER, sizeof(signed char), 1, output); } void unzigzag_iquant(double data[8][8], FILE *input) { signed char to_read[8][8]; // this is the data just read from the input file signed char temp_value = 0; // this is just the temporary value being read from the file // set all the values in the array to zeroes for (unsigned char i = 0; i < 64; i++) to_read[i/8][i%8] = 0; // read from file byte by byte, compare with the marker, and if it is not the marker // and it is not the end of file, read the next byte and store it in the array // keep doing this until the end of block is reached fread(&temp_value, sizeof(signed char), 1, input); for (unsigned char i = 0; (!(feof(input))) && temp_value != MARKER; i++, fread(&temp_value, sizeof(signed char), 1, input)) to_read[i/8][i%8] = temp_value; // unzigzag the temporary array and copy it back to data for (unsigned char i = 0; i < 64; i++) data[i/8][i%8] = (double) to_read[zigzag_tbl[i]/8][zigzag_tbl[i]%8] * quant[i/8][i%8]; }

#### functions.h

#include <stdio.h> #include <math.h> // read the header of the bitmap and write it to the output file void write_read_header(FILE *in, FILE *out); // make the cosine table void make_cosine_tbl(double cosine[8][8]); // zigzag the quantized input data void zigzag_quant(double data[8][8], FILE *output); // unzigzag the zigzagged input data void unzigzag_iquant(double data[8][8], FILE *input);