Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE838 |
| Course Title | SOC Design |
| Semester/Year | W2026 |
| Instructor | Gul N. Khan |

| Lab/Tutorial Report NO. | Lab #3 Report |
| --- | --- |

| Report Title | Lab #3: DE1-SoC Tutorial |
| --- | --- |

| Section No. | 03 |
| --- | --- |
| Submission Date | March 9th, 2025 |
| Due Date | March 10th, 2025 (Extended by 7 days from Mar 3)) |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Abstract/Introduction

This lab experiment introduces design flow of a HW/SW SoC using the DE1-SoC board. The board uses the Cyclone V SoC FPGA. This integrates a Hard Processor System (HPS) based on a dual-core ARM Cortex-A9 processor with FPGA programmable logic.

The objective of this lab is to design a custom FPGA hardware subsystem using Quartus and QSys. It is required to interface the FPGA’s LED, switches, and HEX displays to HPS. Additionally, an embedded systemC software running on Yocto will be used to control the FPGA HW through memory-mapped I/O. Finally, the system will demonstrate bidirectional HPS-FPGA communication using the lightweight HPS-to-FPGA bridge.

The overall system uses 10 LEDs, switches, as well as six 7-segment HEX displays. The software performs an initial light show and then enables real-time switch-controlled LED functionality.

## Methodology

Hardware:

First, a new Quartus project is created targeting the Cyclone V device (5CSEMA5F31C6). The top-level VHDL file (LED_HEX_FPGA.vhd) was added and set as the top-level entity. The given RTL and IP folders are used in the project directory.

Using QSys, the base SoC system (HPS subsystem) was opened.

The IP cores were added:

SEG7_IF (6 HEX displays)

PIO (Parallel I/O) for 10 LEDs (Output)

PIO (Parallel I/O) for 10 Switches (Input)

Component connections:

clk_0

clk_reset

hps_0.h2f_lw_axi_master (Lightweight HPS-to-FPGA bridge)

Conduits exported for:

LED external connection

Switch external connection

SEG7 interface

To prevent overlap, the base addresses were auto-assigned. The system was generated in VHDL, while the .qip file was added to Quartus. Header files were generated using “sopc-create-header-files” (provided memory base addresses for use in C software). [1]

Software:

Using DS-5 with GCC toolchain, the software component was created. Pthread, m, and rt were all the libraries that were linked as well as the path “/usr/local/Quartus-EDS-14.0/embedded/ip/altera/hps/altera_hps/hwlib/include”. 

The program used FPGA peripherals by “#define LW_BRIDGE_BASE 0xFF200000”, with the functions:

alt_write_word()

alt_read_word()

alt_setbits_word()

alt_clrbits_word()

LEDs are accordingly updated while the switches are read continuously.

When the program is executed (launcher HexLEDsSW), a light show sequence runs (LEDs rotate left and right, HEX displays cycle through numbers). After completion, the HEX displays the message: “READY”. In normal mode each switch controls its corresponding LED and the serial terminal prints system state and switch values. [1]

## Results

Figure 1: Altera Eclipse for DS-5 (Terminal Output)

Figure 2: Board Output

Figure 3: Qsys Soc_System File

## Challenges, Outstanding Work, and Future Improvements

Some challenges encountered during the lab are, base address conflicts in QSys, ensuring correct mapping conduit exports, debugging memory mapping alignment issues, as well as configuring SDRAM pins correctly.

To improve this project further, one can add interrupt-based switches, multiple system modules, PWM brightness for LED control, or use FPGA-to-HPS bridge for advanced data transfer.

## Discussion/Conclusion

This lab is a successful demonstration of HW/SW co-design using the SoC Cyclone V software. This lab gives amazing experience in HPS-FPGA bridge communication, custom Avalon slave peripherals, memory-mapped base addresses, compiling and executing Linux applications, and FPGA HW control.

This lab establishes key skills needed for SoC application development, as well as future FPGA-accelerated computing applications.

## References

[1] A. Tino, DE1-SoC Tutorial: COE838 Systems-on-Chip Design Lab 3, Dept. of Electrical, Computer, and Biomedical Engineering, Toronto Metropolitan University, Toronto, ON, Canada, 2024.

## Appendix

#### Main.c

#include <stdio.h> #include <stdlib.h> #include <unistd.h> #include <fcntl.h> #include <time.h> #include <sys/mman.h> #include <stdbool.h> #include <pthread.h> #include "hwlib.h" #include "socal/socal.h" #include "socal/hps.h" #include "socal/alt_gpio.h" #include "hps_0.h" #include "led.h" #include "seg7.h" #define LW_SIZE 0x00200000 #define LWHPS2FPGA_BASE 0xff200000 volatile uint32_t *h2p_lw_led_addr = NULL; volatile uint32_t *h2p_lw_hex_addr = NULL; void led_blink(void){ int i=0; while(1){ printf("LED ON \r\n"); for(i=0;i<=10;i++){ LEDR_LightCount(i); usleep(100*1000); } printf("LED OFF \r\n"); for(i=0;i<=10;i++){ LEDR_OffCount(i); usleep(100*1000); } } } int main(int argc, char **argv){ pthread_t id; int ret; void *virtual_base; int fd; //open the /dev/mem to access the FPGA space for reading and writing if( ( fd = open( "/dev/mem", ( O_RDWR ¦ O_SYNC ) ) ) == -1 ) { printf( "ERROR: could not open \"/dev/mem\"...\n" ); return( 1 ); } //map the virtual memory space to virtual_base, that is 2MB in size (0x00200000), at address LWHPS2FPGA_BASE virtual_base = mmap( NULL, LW_SIZE, ( PROT_READ ¦ PROT_WRITE ), MAP_SHARED, fd, LWHPS2FPGA_BASE); //check that the mapping was successful if( virtual_base == MAP_FAILED ) { printf( "ERROR: mmap() failed...\n" ); close( fd ); return(1); } // map the address space for the LED and HEX registers into user space so we can interact with them. // i.e. the address exists at virtual_base + the offset of your IP component h2p_lw_led_addr= virtual_base + ((uint32_t)(LED_PIO_BASE)); h2p_lw_hex_addr= virtual_base + ( (uint32_t)(SEG7_IF_0_BASE)); //create and run the thread for the LED ret=pthread_create(&id,NULL,(void *)led_blink,NULL); if(ret!=0){ printf("Creat pthread error!\n"); exit(1); } //and run the SEVEN SEG process SEG7_All_Number(); //once the SEG7 show is complete, display a message display_msg(); pthread_join(id,NULL); if( munmap( virtual_base, LW_SIZE) != 0 ) { printf( "ERROR: munmap() failed...\n" ); close( fd ); return( 1 ); } close( fd ); return 0; }

#### led.c

#include "led.h" #include "hwlib.h" #include "socal/socal.h" #include "socal/hps.h" #include "socal/alt_gpio.h" extern volatile unsigned long *h2p_lw_led_addr; void LEDR_LightCount(unsigned char LightCount){ // 1: light, 0:unlight uint32_t Mask = 0; int i; for(i=0;i<LightCount;i++){ Mask <<= 1; Mask ¦= 0x01; } alt_write_word(h2p_lw_led_addr, Mask ); //0:ligh, 1:unlight } void LEDR_OffCount(unsigned char OffCount){ // 1: light, 0:unlight uint32_t Mask = 0x03ff; int i; for(i=0;i<OffCount;i++){ Mask >>= 1; } alt_write_word(h2p_lw_led_addr, Mask ); //0:ligh, 1:unlight } void LEDR_AllOn(void){ alt_write_word(h2p_lw_led_addr, 0x3FF); //0:ligh, 1:unlight } void LEDR_AllOff(void){ alt_write_word(h2p_lw_led_addr, 0x00); //0:ligh, 1:unlight }

#### led.h

#ifndef LED_H_ #define LED_H_ void LEDR_LightCount(unsigned char LightCount); void LEDR_OffCount(unsigned char OffCount); void LEDR_AllOn(void); void LEDR_AllOff(void);

#### 7seg.c

#include <stdio.h> #include <unistd.h> #include "seg7.h" #include "hwlib.h" #include "socal/socal.h" #include "socal/hps.h" #include "socal/alt_gpio.h" extern volatile unsigned long *h2p_lw_hex_addr; #define SEG7_SET(index, seg_mask) alt_write_word(h2p_lw_hex_addr+index,seg_mask) #define SEG7_NUM 6 static unsigned char szMap[] = { 63, 6, 91, 79, 102, 109, 125, 7, 127, 111, 119, 124, 57, 94, 121, 113 }; // 0,1,2, ... 9, a, b, c, d, e, f static unsigned char msg[] = { 63, 56, 56, 121, 118, 0 }; //HELLO (OLLEH) void display_msg(){ SEG7_SET(0, msg[0]); SEG7_SET(1, msg[1]); SEG7_SET(2, msg[2]); SEG7_SET(3, msg[3]); SEG7_SET(4, msg[4]); SEG7_SET(5, msg[5]); } void SEG7_Clear(void){ int i; for(i=0;i<SEG7_NUM;i++){ SEG7_SET(i, 0x00); } } void SEG7_Full(void){ int i; for(i=0;i<SEG7_NUM;i++){ SEG7_SET(i, 0xFF); } } void SEG7_Number(void){ int i; for(i=0;i<SEG7_NUM;i++){ SEG7_SET(i, szMap[i]); } } void SEG7_All_Number(void){ int i,j; for(j=0;j<16;j++) { printf("hex show %X\r\n",j); for(i=0;i<SEG7_NUM;i++){ SEG7_SET(i, szMap[j]); } usleep(500*1000); } } void SEG7_Hex( unsigned long Data,unsigned char point_mask){ unsigned char mask = 0x01; unsigned char seg_mask; int i; // seg_mask = 0; for(i=0;i<SEG7_NUM;i++){ seg_mask = szMap[Data & 0x0F]; Data >>= 4; if (point_mask & mask) seg_mask ¦= 0x80; mask <<= 1; SEG7_SET(i, seg_mask); } } void SEG7_Decimal( unsigned long Data, unsigned char point_mask){ unsigned char mask = 0x01; unsigned char seg_mask; int i; // seg_mask = 0; for(i=0;i<SEG7_NUM;i++){ seg_mask = szMap[Data%10]; Data /= 10; if (point_mask & mask) seg_mask ¦= 0x80; mask <<= 1; SEG7_SET(i, seg_mask); } }

#### 7seg.h

#ifndef SEG7_H_ #define SEG7_H_ void SEG7_Clear(void); void SEG7_Full(void); void SEG7_Hex( unsigned long Data, unsigned char point_mask); void SEG7_Decimal( unsigned long Data, unsigned char point_mask); void SEG7_Number(void); void SEG7_All_Number(void); void display_msg(); #endif /* SEG7_H_ */

#### hps_0.h

#define SEG7_IF_0_COMPONENT_TYPE SEG7_IF #define SEG7_IF_0_COMPONENT_NAME SEG7_IF_0 #define SEG7_IF_0_BASE 0x0 #define SEG7_IF_0_SPAN 64 #define SEG7_IF_0_END 0x3f /* * Macros for device 'sw_pio', class 'altera_avalon_pio' * The macros are prefixed with 'SW_PIO_'. * The prefix is the slave descriptor. */ #define SW_PIO_COMPONENT_TYPE altera_avalon_pio #define SW_PIO_COMPONENT_NAME sw_pio #define SW_PIO_BASE 0x20 #define SW_PIO_SPAN 32 #define SW_PIO_END 0x3f #define SW_PIO_BIT_CLEARING_EDGE_REGISTER 0 #define SW_PIO_BIT_MODIFYING_OUTPUT_REGISTER 0 #define SW_PIO_CAPTURE 0 #define SW_PIO_DATA_WIDTH 10 #define SW_PIO_DO_TEST_BENCH_WIRING 0 #define SW_PIO_DRIVEN_SIM_VALUE 0 #define SW_PIO_EDGE_TYPE NONE #define SW_PIO_FREQ 50000000 #define SW_PIO_HAS_IN 1 #define SW_PIO_HAS_OUT 0 #define SW_PIO_HAS_TRI 0 #define SW_PIO_IRQ_TYPE NONE #define SW_PIO_RESET_VALUE 0 /* * Macros for device 'led_pio', class 'altera_avalon_pio' * The macros are prefixed with 'LED_PIO_'. * The prefix is the slave descriptor. */ #define LED_PIO_COMPONENT_TYPE altera_avalon_pio #define LED_PIO_COMPONENT_NAME led_pio #define LED_PIO_BASE 0x30 #define LED_PIO_SPAN 32 #define LED_PIO_END 0x4f #define LED_PIO_BIT_CLEARING_EDGE_REGISTER 0 #define LED_PIO_BIT_MODIFYING_OUTPUT_REGISTER 0 #define LED_PIO_CAPTURE 0 #define LED_PIO_DATA_WIDTH 10 #define LED_PIO_DO_TEST_BENCH_WIRING 0 #define LED_PIO_DRIVEN_SIM_VALUE 0 #define LED_PIO_EDGE_TYPE NONE #define LED_PIO_FREQ 50000000 #define LED_PIO_HAS_IN 0 #define LED_PIO_HAS_OUT 1 #define LED_PIO_HAS_TRI 0 #define LED_PIO_IRQ_TYPE NONE #define LED_PIO_RESET_VALUE 0 #endif /* _ALTERA_HPS_0_H_ */

#### LED_HEX_FPGA.vhd

----------------------------- -- HED_LED_FPGA Tutorial -- Lab 3 -- COE838 Systems-on-Chip Design ----------------------------- library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; ENTITY LED_HEX_FPGA IS PORT( CLOCK_50, HPS_DDR3_RZQ,HPS_ENET_RX_CLK, HPS_ENET_RX_DV : IN STD_LOGIC; HPS_DDR3_ADDR : OUT STD_LOGIC_VECTOR(14 DOWNTO 0); HPS_DDR3_BA : OUT STD_LOGIC_VECTOR(2 DOWNTO 0); HPS_DDR3_CS_N : OUT STD_LOGIC; HPS_DDR3_CK_P, HPS_DDR3_CK_N, HPS_DDR3_CKE : OUT STD_LOGIC; HPS_USB_DIR, HPS_USB_NXT, HPS_USB_CLKOUT : IN STD_LOGIC; HPS_ENET_RX_DATA : IN STD_LOGIC_VECTOR(3 DOWNTO 0); HPS_SD_DATA, HPS_DDR3_DQS_N, HPS_DDR3_DQS_P : INOUT STD_LOGIC_VECTOR(3 DOWNTO 0); HPS_ENET_MDIO : INOUT STD_LOGIC; HPS_USB_DATA : INOUT STD_LOGIC_VECTOR(7 DOWNTO 0); HPS_DDR3_DQ : INOUT STD_LOGIC_VECTOR(31 DOWNTO 0); HPS_SD_CMD : INOUT STD_LOGIC; HPS_ENET_TX_DATA, HPS_DDR3_DM : OUT STD_LOGIC_VECTOR(3 DOWNTO 0); HPS_DDR3_ODT, HPS_DDR3_RAS_N, HPS_DDR3_RESET_N : OUT STD_LOGIC; HPS_DDR3_CAS_N, HPS_DDR3_WE_N : OUT STD_LOGIC; HPS_ENET_MDC, HPS_ENET_TX_EN : OUT STD_LOGIC; LEDR : OUT STD_LOGIC_VECTOR(9 DOWNTO 0); HEX0, HEX1, HEX2, HEX3, HEX4, HEX5 : BUFFER STD_LOGIC_VECTOR(6 DOWNTO 0); HPS_USB_STP, HPS_SD_CLK, HPS_ENET_GTX_CLK : OUT STD_LOGIC; SW : IN STD_LOGIC_VECTOR(9 DOWNTO 0)); END LED_HEX_FPGA; ARCHITECTURE Behaviour OF LED_HEX_FPGA IS --instantiate the soc_systtem component here component soc_system is port ( clk_clk : in std_logic := 'X'; -- clk hps_0_h2f_reset_reset_n : out std_logic; -- reset_n hps_io_hps_io_emac1_inst_TX_CLK : out std_logic; -- hps_io_emac1_inst_TX_CLK hps_io_hps_io_emac1_inst_TXD0 : out std_logic; -- hps_io_emac1_inst_TXD0 hps_io_hps_io_emac1_inst_TXD1 : out std_logic; -- hps_io_emac1_inst_TXD1 hps_io_hps_io_emac1_inst_TXD2 : out std_logic; -- hps_io_emac1_inst_TXD2 hps_io_hps_io_emac1_inst_TXD3 : out std_logic; -- hps_io_emac1_inst_TXD3 hps_io_hps_io_emac1_inst_RXD0 : in std_logic := 'X'; -- hps_io_emac1_inst_RXD0 hps_io_hps_io_emac1_inst_MDIO : inout std_logic := 'X'; -- hps_io_emac1_inst_MDIO hps_io_hps_io_emac1_inst_MDC : out std_logic; -- hps_io_emac1_inst_MDC hps_io_hps_io_emac1_inst_RX_CTL : in std_logic := 'X'; -- hps_io_emac1_inst_RX_CTL hps_io_hps_io_emac1_inst_TX_CTL : out std_logic; -- hps_io_emac1_inst_TX_CTL hps_io_hps_io_emac1_inst_RX_CLK : in std_logic := 'X'; -- hps_io_emac1_inst_RX_CLK hps_io_hps_io_emac1_inst_RXD1 : in std_logic := 'X'; -- hps_io_emac1_inst_RXD1 hps_io_hps_io_emac1_inst_RXD2 : in std_logic := 'X'; -- hps_io_emac1_inst_RXD2 hps_io_hps_io_emac1_inst_RXD3 : in std_logic := 'X'; -- hps_io_emac1_inst_RXD3 hps_io_hps_io_sdio_inst_CMD : inout std_logic := 'X'; -- hps_io_sdio_inst_CMD hps_io_hps_io_sdio_inst_D0 : inout std_logic := 'X'; -- hps_io_sdio_inst_D0 hps_io_hps_io_sdio_inst_D1 : inout std_logic := 'X'; -- hps_io_sdio_inst_D1 hps_io_hps_io_sdio_inst_CLK : out std_logic; -- hps_io_sdio_inst_CLK hps_io_hps_io_sdio_inst_D2 : inout std_logic := 'X'; -- hps_io_sdio_inst_D2 hps_io_hps_io_sdio_inst_D3 : inout std_logic := 'X'; -- hps_io_sdio_inst_D3 hps_io_hps_io_usb1_inst_D0 : inout std_logic := 'X'; -- hps_io_usb1_inst_D0 hps_io_hps_io_usb1_inst_D1 : inout std_logic := 'X'; -- hps_io_usb1_inst_D1 hps_io_hps_io_usb1_inst_D2 : inout std_logic := 'X'; -- hps_io_usb1_inst_D2 hps_io_hps_io_usb1_inst_D3 : inout std_logic := 'X'; -- hps_io_usb1_inst_D3 hps_io_hps_io_usb1_inst_D4 : inout std_logic := 'X'; -- hps_io_usb1_inst_D4 hps_io_hps_io_usb1_inst_D5 : inout std_logic := 'X'; -- hps_io_usb1_inst_D5 hps_io_hps_io_usb1_inst_D6 : inout std_logic := 'X'; -- hps_io_usb1_inst_D6 hps_io_hps_io_usb1_inst_D7 : inout std_logic := 'X'; -- hps_io_usb1_inst_D7 hps_io_hps_io_usb1_inst_CLK : in std_logic := 'X'; -- hps_io_usb1_inst_CLK hps_io_hps_io_usb1_inst_STP : out std_logic; -- hps_io_usb1_inst_STP hps_io_hps_io_usb1_inst_DIR : in std_logic := 'X'; -- hps_io_usb1_inst_DIR hps_io_hps_io_usb1_inst_NXT : in std_logic := 'X'; -- hps_io_usb1_inst_NXT memory_mem_a : out std_logic_vector(14 downto 0); -- mem_a memory_mem_ba : out std_logic_vector(2 downto 0); -- mem_ba memory_mem_ck : out std_logic; -- mem_ck memory_mem_ck_n : out std_logic; -- mem_ck_n memory_mem_cke : out std_logic; -- mem_cke memory_mem_cs_n : out std_logic; -- mem_cs_n memory_mem_ras_n : out std_logic; -- mem_ras_n memory_mem_cas_n : out std_logic; -- mem_cas_n memory_mem_we_n : out std_logic; -- mem_we_n memory_mem_reset_n : out std_logic; -- mem_reset_n memory_mem_dq : inout std_logic_vector(31 downto 0) := (others => 'X'); -- mem_dq memory_mem_dqs : inout std_logic_vector(3 downto 0) := (others => 'X'); -- mem_dqs memory_mem_dqs_n : inout std_logic_vector(3 downto 0) := (others => 'X'); -- mem_dqs_n memory_mem_odt : out std_logic; -- mem_odt memory_mem_dm : out std_logic_vector(3 downto 0); -- mem_dm memory_oct_rzqin : in std_logic := 'X'; -- oct_rzqin reset_reset_n : in std_logic := 'X'; -- reset_n led_pio_external_connection_export : out std_logic_vector(9 downto 0); -- export seg7_if_0_conduit_end_export : out std_logic_vector(47 downto 0); -- export sw_pio_external_connection_export : in std_logic_vector(9 downto 0) := (others => 'X') -- export ); end component soc_system; --SIGNALS instantiated here SIGNAL hex5_tmp, hex4_tmp, hex3_tmp, hex2_tmp, hex1_tmp, hex0_tmp, hps_0_h2f_reset_reset_n : STD_LOGIC; BEGIN --port map soc_system here u0 : component soc_system port map ( clk_clk => CLOCK_50, reset_reset_n => '1',memory_mem_a => HPS_DDR3_ADDR, memory_mem_ba => HPS_DDR3_BA, memory_mem_ck => HPS_DDR3_CK_P, memory_mem_ck_n => HPS_DDR3_CK_N, memory_mem_cke => HPS_DDR3_CKE, memory_mem_cs_n => HPS_DDR3_CS_N, memory_mem_ras_n => HPS_DDR3_RAS_N, memory_mem_cas_n => HPS_DDR3_CAS_N, memory_mem_we_n => HPS_DDR3_WE_N, memory_mem_reset_n => HPS_DDR3_RESET_N, memory_mem_dq => HPS_DDR3_DQ, memory_mem_dqs => HPS_DDR3_DQS_P, memory_mem_dqs_n => HPS_DDR3_DQS_N, memory_mem_odt => HPS_DDR3_ODT, memory_mem_dm => HPS_DDR3_DM, memory_oct_rzqin => HPS_DDR3_RZQ, hps_io_hps_io_emac1_inst_TX_CLK => HPS_ENET_GTX_CLK, hps_io_hps_io_emac1_inst_TXD0 => HPS_ENET_TX_DATA(0), hps_io_hps_io_emac1_inst_TXD1 => HPS_ENET_TX_DATA(1), hps_io_hps_io_emac1_inst_TXD2 => HPS_ENET_TX_DATA(2), hps_io_hps_io_emac1_inst_TXD3 => HPS_ENET_TX_DATA(3), hps_io_hps_io_emac1_inst_RXD0 => HPS_ENET_RX_DATA(0), hps_io_hps_io_emac1_inst_MDIO => HPS_ENET_MDIO, hps_io_hps_io_emac1_inst_MDC => HPS_ENET_MDC, hps_io_hps_io_emac1_inst_RX_CTL => HPS_ENET_RX_DV, hps_io_hps_io_emac1_inst_TX_CTL => HPS_ENET_TX_EN, hps_io_hps_io_emac1_inst_RX_CLK => HPS_ENET_RX_CLK, hps_io_hps_io_emac1_inst_RXD1 => HPS_ENET_RX_DATA(1), hps_io_hps_io_emac1_inst_RXD2 => HPS_ENET_RX_DATA(2), hps_io_hps_io_emac1_inst_RXD3 => HPS_ENET_RX_DATA(3), hps_io_hps_io_sdio_inst_CMD => HPS_SD_CMD, hps_io_hps_io_sdio_inst_D0 => HPS_SD_DATA(0), hps_io_hps_io_sdio_inst_D1 => HPS_SD_DATA(1), hps_io_hps_io_sdio_inst_CLK => HPS_SD_CLK, hps_io_hps_io_sdio_inst_D2 => HPS_SD_DATA(2), hps_io_hps_io_sdio_inst_D3 => HPS_SD_DATA(3), hps_io_hps_io_usb1_inst_D0 => HPS_USB_DATA(0), hps_io_hps_io_usb1_inst_D1 => HPS_USB_DATA(1), hps_io_hps_io_usb1_inst_D2 => HPS_USB_DATA(2), hps_io_hps_io_usb1_inst_D3 => HPS_USB_DATA(3), hps_io_hps_io_usb1_inst_D4 => HPS_USB_DATA(4), hps_io_hps_io_usb1_inst_D5 => HPS_USB_DATA(5), hps_io_hps_io_usb1_inst_D6 => HPS_USB_DATA(6), hps_io_hps_io_usb1_inst_D7 => HPS_USB_DATA(7), hps_io_hps_io_usb1_inst_CLK => HPS_USB_CLKOUT, hps_io_hps_io_usb1_inst_STP => HPS_USB_STP, hps_io_hps_io_usb1_inst_DIR => HPS_USB_DIR,hps_io_hps_io_usb1_inst_NXT => HPS_USB_NXT, hps_0_h2f_reset_reset_n => hps_0_h2f_reset_reset_n, led_pio_external_connection_export => LEDR, seg7_if_0_conduit_end_export(47) => hex5_tmp, seg7_if_0_conduit_end_export(46 DOWNTO 40)=>HEX5, seg7_if_0_conduit_end_export(39) =>hex4_tmp, seg7_if_0_conduit_end_export(38 DOWNTO 32)=>HEX4, seg7_if_0_conduit_end_export(31) =>hex3_tmp, seg7_if_0_conduit_end_export(30 DOWNTO 24)=>HEX3, seg7_if_0_conduit_end_export(23) =>hex2_tmp, seg7_if_0_conduit_end_export(22 DOWNTO 16)=> HEX2, seg7_if_0_conduit_end_export(15) => hex1_tmp, seg7_if_0_conduit_end_export(14 DOWNTO 8) => HEX1, seg7_if_0_conduit_end_export(7) => hex0_tmp, seg7_if_0_conduit_end_export(6 DOWNTO 0) =>HEX0, sw_pio_external_connection_export => SW ); End Behaviour;