Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE838 |
| Course Title | SOC Design |
| Semester/Year | W2026 |
| Instructor | Gul N. Khan |

| Lab/Tutorial Report NO. | Lab #4 Report |
| --- | --- |

| Report Title | Lab #4: Interfacing Custom IP Cores to HPS/FPGA SOC Platform |
| --- | --- |

| Section No. | 03 |
| --- | --- |
| Submission Date | March 9th, 2025 |
| Due Date | March 10th, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Abstract/Introduction

Modern SoC platforms combine embedded processors and programmable logic in order to make HW/SW systems that are very flexible. This lab aims to teach how to integrate Intellectual Property (IP) cores into a HPS/FPGA system while allowing communication between hardware (HW) and software (SW) through memory-mapped interfaces.

A custom multiplier IP core was made to a Cyclone V SoC design by using Quartus and Qsys. Avalon Memory-Mapped (Avalon-MM) slave interfaces were designed to allow the Hard Processor System (HPS) to communicate to the FPGA. There were two Avalon slave components: control signals, and data transfer components. A wrapper module was used for the multiplexer execution as well as to generate a completion signal after a fixed latency.

A C program running on HPS accessed the IP through memory mapping registers to complete multiplication and to verify correctness.

## Methodology

Hardware:

A Quartus project named “custom_ip” was made in Cyclone-V 5CSEMA5F31C6 device used on the DE1-SoC board. A VHDL file (top-level) called “custom_ip.vhdl was created. This entity instantiated “soc_system” and “mult_unit”. After, a .sof file was made to program the FPGA.

This integrates:

- Hard Processor System (HPS)

- Custom multiplier IP

- Avalon-MM slave interfaces

- Wrapper logic for multiplier control

A multiplier IP core was made using the IP Catalog with Quartus. This was saved as “mult.vhd” [1].

Table 1. Configuration Parameters of the IP Core [1]

| Parameter | Value |
| --- | --- |
| Input A width | 16 bits |
| Input B width | 16 bits |
| Result width | 32 bits |
| Signed/Unsigned | Unsigned |
| Output latency | 3 clock cycles |
| Clock enable | Enabled |
| Synchronous clear | Enabled |

This multiplier completes the following calculation:

Result=dataa×datab [1] [equation 1] (3-cycle pipeline latency)

A wrapper module called “mult_unit.vhd” is made to ensure three clock cycles to complete computation. The wrapper accepts input operands from the Avalon data interface, enables the multiplier, counts the three clock cycles, and outputs a done flag once the result is ready.

Two Avalon-MM slave modules (mult_control, mult_data) are made to control reset, done, and start signals as well as transfer operands and results respectively. These components were connected to the HPS lightweight AXI master, system clock and reset
, and also exported conduits to the multiplier wrapper
The SoC architecture therefore included: an HPS processor, Avalon interconnect, control slave, data slave as well as a custom multiplier IP. [1].

Software:

A C program was created to control the hardware multiplier from the HPS.

The program completes the following:

- Map FPGA registers into HPS virtual memory.

- Reset the multiplier.

- Write operand values to the data registers.

- Trigger the start signal.

- Poll the done flag.

- Read the multiplication result.

- Compare the hardware result with a software calculation [1].

## Results

Figure 1. Terminal Output

Figure 2. Qsys File

A successful run can be seen due to the following output from the terminal:
[MULT TEST PASSED] 256/256

[DIV TEST PASSED] 256/256

## Challenges, Outstanding Work, and Future Improvements

Some challenges that were encountered during the implementation of the project were that QSys initially misinterpreted some ports in the VHDL files. These signals had to be assigned manually as conduit interfaces rather than Avalon Signals. Also, wrong offsets in the C code allowed for read/write errors until the correct addresses were verified using the hps_0.h file.

Some future improvements that can be made are supporting larger operand widths (e.g., 32-bit multiplication), implementing interrupts instead of polling for the done signal, adding additional arithmetic IP cores (e.g., divider or ALU), or creating a more complex hardware accelerator for signal processing tasks.

## Discussion/Conclusion

This lab successfully demonstrated a custom IP core that can be integrated into a HPS/FPGA SoC system with software control. Some key SoC techniques include: generating custom IP cores using Quartus, designing Avalon Memory-Mapped slave interfaces, integrating hardware components using QSys, establishing communication between FPGA logic and the HPS processor, and developing software to control hardware accelerators. A proper execution of this design confirms that the custom IP core was correctly designed and implemented. This methodology is commonly used in embedded systems where compute-intensive operations are required and offloaded to hardware accelerators for performance improvements.

## References

[1] Department of Electrical, Computer and Biomedical Engineering,

"Lab 4: Interfacing Custom IP Cores to HPS/FPGA SoC Platform,"

COE838: Systems-on-Chip Design Lab Manual, Toronto Metropolitan University, Toronto, ON, Canada.

## Appendix

### main.c

#include <stdio.h> #include <stdlib.h> #include <unistd.h> #include <fcntl.h> #include <time.h> #include <sys/mman.h> #include "hwlib.h" #include "socal/socal.h" #include "socal/hps.h" #include "socal/alt_gpio.h" #include "hps_0.h" #define LW_SIZE 0x00200000 #define LWHPS2FPGA_BASE 0xff200000 volatile uint32_t *mult_control = NULL; volatile uint32_t *mult_data = NULL; volatile uint32_t *div_control = NULL; volatile uint32_t *div_data = NULL; int mult_success, mult_total; int div_success, div_total; void reset_system(){ alt_write_word(mult_control+1, 0x1); //assert mult reset while(!(alt_read_word(mult_control+1) & 0x1)); printf("MULT Reset done. Deasserting signal\n"); while((alt_read_word(mult_control+1) & 0x1));//deassert mult reset alt_write_word(div_control+1, 0x1); //assert div reset while(!(alt_read_word(div_control+1) & 0x1)); printf("DIV Reset done. Deasserting signal\n"); while((alt_read_word(div_control+1) & 0x1));//deassert div reset } void copy_to_input(uint32_t a, uint32_t b){ alt_write_word(mult_data, a); //write input A to MULT unit alt_write_word(mult_data+1, b); //write input B to MULT unit //start conversion alt_write_word(mult_control, 0x00000001); //double check that div start was asserted while(!(alt_read_word(mult_control) & 0x1)); printf("MULT Start successful\n"); alt_write_word(div_data, b); //write denominator to DIV unit alt_write_word(div_data+1, a); //write numerator to DIV unit //start conversion alt_write_word(div_control, 0x00000001); //write 1 to DIV start //double check that div start was asserted while(!(alt_read_word(div_control) & 0x1)); printf("DIV Start successful\n"); } void copy_output(){ uint32_t word, op1, op2; //wait for MULT done printf("waiting for MULT done\n"); while(!(alt_read_word(mult_control+2) & 0x1)); printf("MULT conversion done\n"); word = alt_read_word(mult_data+0); op1 = alt_read_word(mult_data+1); op2 = alt_read_word(mult_data+2); printf("0x%08x * 0x%08x = 0x%08x. [Expected] 0x%08x\n", op1, op2, word, (op1*op2)); if(word == (op1*op2)){ printf("[SUCCESSFUL]\n"); mult_success++; }else{ printf("[FAILED]\n"); } mult_total++; printf("------\n"); uint32_t quotient, remainder, denominator, numerator; uint32_t expected_q, expected_r; //wait for DIV done printf("waiting for DIV done\n"); while(!(alt_read_word(div_control+2) & 0x1)); printf("DIV conversion done\n"); quotient = alt_read_word(div_data+0); remainder = alt_read_word(div_data+1); denominator = alt_read_word(div_data+2); numerator = alt_read_word(div_data+3); if(denominator == 0){ expected_q = 0x0000FFFF; //max value for 16 bit word expected_r = numerator; } else{ expected_q = ((uint16_t)numerator) / ((uint16_t)denominator); expected_r = numerator % denominator; } printf("0x%08x / 0x%08x = 0x%08x + r0x%08x. [Expected] 0x%08x + r0x%08x\n", numerator, denominator, quotient, remainder, expected_q, expected_r); if((quotient == expected_q) && (remainder == expected_r)){ printf("[SUCCESSFUL]\n"); div_success++; }else{ printf("[FAILED]\n"); } div_total++; printf("------------------------------------------------\n"); } int main(int argc, char **argv){ int fd, i, j; void *virtual_base; mult_success = 0; mult_total = 0; //map address space of fpga for software to access here if((fd = open("/dev/mem", ( O_RDWR ¦ O_SYNC ) ) ) == -1 ) { printf( "ERROR: could not open \"/dev/mem\"...\n" ); return( 1 ); } virtual_base = mmap( NULL, LW_SIZE, ( PROT_READ ¦ PROT_WRITE ), MAP_SHARED, fd, LWHPS2FPGA_BASE); if( virtual_base == MAP_FAILED ) { printf( "ERROR: mmap() failed...\n" ); close( fd ); return(1); } //initialize the addresses mult_control = virtual_base + ((uint32_t)(MULT_CONTROL_0_BASE)); mult_data = virtual_base + ((uint32_t)(MULT_DATA_0_BASE)); div_control = virtual_base + ((uint32_t)(DIV_CONTROL_0_BASE)); div_data = virtual_base + ((uint32_t)(DIV_DATA_0_BASE)); printf("------>Finished initializing HPS/FPGA system<-------\n"); for(i = 0; i < 16; i++){ for(j = 0; j < 16; j++){ printf("---------------- Iteration [%d, %d] ------------------\n", i, j); reset_system(); copy_to_input((uint32_t)i, (uint32_t)j); copy_output(); } } printf("[MULT TEST PASSED] %d/%d\n", mult_success, mult_total); printf("[DIV TEST PASSED] %d/%d\n", div_success, div_total); // clean up our memory mapping and exit if( munmap( virtual_base, LW_SIZE) != 0 ) { printf( "ERROR: munmap() failed...\n" ); close( fd ); return( 1 ); } close( fd ); return 0; }

### hps_0.h

#ifndef _ALTERA_HPS_0_H_ #define _ALTERA_HPS_0_H_ /* * This file was automatically generated by the swinfo2header utility. * * Created from SOPC Builder system 'soc_system' in * file './soc_system.sopcinfo'. */ /* * This file contains macros for module 'hps_0' and devices * connected to the following masters: * h2f_axi_master * h2f_lw_axi_master * * Do not include this header file and another header file created for a * different module or master group at the same time. * Doing so may result in duplicate macro names. * Instead, use the system header file which has macros with unique names. */ /* * Macros for device 'div_data_0', class 'div_data' * The macros are prefixed with 'DIV_DATA_0_'. * The prefix is the slave descriptor. */ #define DIV_DATA_0_COMPONENT_TYPE div_data #define DIV_DATA_0_COMPONENT_NAME div_data_0 #define DIV_DATA_0_BASE 0x0 #define DIV_DATA_0_SPAN 64 #define DIV_DATA_0_END 0x3f /* * Macros for device 'div_control_0', class 'div_control' * The macros are prefixed with 'DIV_CONTROL_0_'. * The prefix is the slave descriptor. */ #define DIV_CONTROL_0_COMPONENT_TYPE div_control #define DIV_CONTROL_0_COMPONENT_NAME div_control_0 #define DIV_CONTROL_0_BASE 0x40 #define DIV_CONTROL_0_SPAN 64 #define DIV_CONTROL_0_END 0x7f /* * Macros for device 'mult_control_0', class 'mult_control' * The macros are prefixed with 'MULT_CONTROL_0_'. * The prefix is the slave descriptor. */ #define MULT_CONTROL_0_COMPONENT_TYPE mult_control #define MULT_CONTROL_0_COMPONENT_NAME mult_control_0 #define MULT_CONTROL_0_BASE 0x80 #define MULT_CONTROL_0_SPAN 64 #define MULT_CONTROL_0_END 0xbf /* * Macros for device 'mult_data_0', class 'mult_data' * The macros are prefixed with 'MULT_DATA_0_'. * The prefix is the slave descriptor. */ #define MULT_DATA_0_COMPONENT_TYPE mult_data #define MULT_DATA_0_COMPONENT_NAME mult_data_0 #define MULT_DATA_0_BASE 0xc0 #define MULT_DATA_0_SPAN 64 #define MULT_DATA_0_END 0xff #endif /* _ALTERA_HPS_0_H_ */

### custom_ip.vhd

library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; ENTITY custom_ip IS PORT( CLOCK_50, HPS_DDR3_RZQ,HPS_ENET_RX_CLK, HPS_ENET_RX_DV : IN STD_LOGIC; HPS_DDR3_ADDR : OUT STD_LOGIC_VECTOR(14 DOWNTO 0); HPS_DDR3_BA : OUT STD_LOGIC_VECTOR(2 DOWNTO 0); HPS_DDR3_CS_N : OUT STD_LOGIC; HPS_DDR3_CK_P, HPS_DDR3_CK_N, HPS_DDR3_CKE : OUT STD_LOGIC; HPS_USB_DIR, HPS_USB_NXT, HPS_USB_CLKOUT : IN STD_LOGIC; HPS_ENET_RX_DATA : IN STD_LOGIC_VECTOR(3 DOWNTO 0); HPS_SD_DATA, HPS_DDR3_DQS_N, HPS_DDR3_DQS_P : INOUT STD_LOGIC_VECTOR(3 DOWNTO 0); HPS_ENET_MDIO : INOUT STD_LOGIC; HPS_USB_DATA : INOUT STD_LOGIC_VECTOR(7 DOWNTO 0); HPS_DDR3_DQ : INOUT STD_LOGIC_VECTOR(31 DOWNTO 0); HPS_SD_CMD : INOUT STD_LOGIC; HPS_ENET_TX_DATA, HPS_DDR3_DM : OUT STD_LOGIC_VECTOR(3 DOWNTO 0); HPS_DDR3_ODT, HPS_DDR3_RAS_N, HPS_DDR3_RESET_N : OUT STD_LOGIC; HPS_DDR3_CAS_N, HPS_DDR3_WE_N : OUT STD_LOGIC; HPS_ENET_MDC, HPS_ENET_TX_EN : OUT STD_LOGIC; HPS_USB_STP, HPS_SD_CLK, HPS_ENET_GTX_CLK : OUT STD_LOGIC); END custom_ip; ARCHITECTURE Behaviour OF custom_ip IS --instantiate the soc_systtem component here component soc_system is port ( clk_clk : in std_logic := 'X'; -- clk hps_0_h2f_reset_reset_n : out std_logic; -- reset_n hps_io_hps_io_emac1_inst_TX_CLK : out std_logic; -- hps_io_emac1_inst_TX_CLK hps_io_hps_io_emac1_inst_TXD0 : out std_logic; -- hps_io_emac1_inst_TXD0 hps_io_hps_io_emac1_inst_TXD1 : out std_logic; -- hps_io_emac1_inst_TXD1 hps_io_hps_io_emac1_inst_TXD2 : out std_logic; -- hps_io_emac1_inst_TXD2 hps_io_hps_io_emac1_inst_TXD3 : out std_logic; -- hps_io_emac1_inst_TXD3 hps_io_hps_io_emac1_inst_RXD0 : in std_logic := 'X'; -- hps_io_emac1_inst_RXD0 hps_io_hps_io_emac1_inst_MDIO : inout std_logic := 'X'; -- hps_io_emac1_inst_MDIO hps_io_hps_io_emac1_inst_MDC : out std_logic; -- hps_io_emac1_inst_MDC hps_io_hps_io_emac1_inst_RX_CTL : in std_logic := 'X'; -- hps_io_emac1_inst_RX_CTL hps_io_hps_io_emac1_inst_TX_CTL : out std_logic; -- hps_io_emac1_inst_TX_CTL hps_io_hps_io_emac1_inst_RX_CLK : in std_logic := 'X'; -- hps_io_emac1_inst_RX_CLK hps_io_hps_io_emac1_inst_RXD1 : in std_logic := 'X'; -- hps_io_emac1_inst_RXD1 hps_io_hps_io_emac1_inst_RXD2 : in std_logic := 'X'; -- hps_io_emac1_inst_RXD2 hps_io_hps_io_emac1_inst_RXD3 : in std_logic := 'X'; -- hps_io_emac1_inst_RXD3 hps_io_hps_io_sdio_inst_CMD : inout std_logic := 'X'; -- hps_io_sdio_inst_CMD hps_io_hps_io_sdio_inst_D0 : inout std_logic := 'X'; -- hps_io_sdio_inst_D0 hps_io_hps_io_sdio_inst_D1 : inout std_logic := 'X'; -- hps_io_sdio_inst_D1 hps_io_hps_io_sdio_inst_CLK : out std_logic; -- hps_io_sdio_inst_CLK hps_io_hps_io_sdio_inst_D2 : inout std_logic := 'X'; -- hps_io_sdio_inst_D2 hps_io_hps_io_sdio_inst_D3 : inout std_logic := 'X'; -- hps_io_sdio_inst_D3 hps_io_hps_io_usb1_inst_D0 : inout std_logic := 'X'; -- hps_io_usb1_inst_D0 hps_io_hps_io_usb1_inst_D1 : inout std_logic := 'X'; -- hps_io_usb1_inst_D1 hps_io_hps_io_usb1_inst_D2 : inout std_logic := 'X'; -- hps_io_usb1_inst_D2 hps_io_hps_io_usb1_inst_D3 : inout std_logic := 'X'; -- hps_io_usb1_inst_D3 hps_io_hps_io_usb1_inst_D4 : inout std_logic := 'X'; -- hps_io_usb1_inst_D4 hps_io_hps_io_usb1_inst_D5 : inout std_logic := 'X'; -- hps_io_usb1_inst_D5 hps_io_hps_io_usb1_inst_D6 : inout std_logic := 'X'; -- hps_io_usb1_inst_D6 hps_io_hps_io_usb1_inst_D7 : inout std_logic := 'X'; -- hps_io_usb1_inst_D7 hps_io_hps_io_usb1_inst_CLK : in std_logic := 'X'; -- hps_io_usb1_inst_CLK hps_io_hps_io_usb1_inst_STP : out std_logic; -- hps_io_usb1_inst_STP hps_io_hps_io_usb1_inst_DIR : in std_logic := 'X'; -- hps_io_usb1_inst_DIR hps_io_hps_io_usb1_inst_NXT : in std_logic := 'X'; -- hps_io_usb1_inst_NXT memory_mem_a : out std_logic_vector(14 downto 0); -- mem_a memory_mem_ba : out std_logic_vector(2 downto 0); -- mem_ba memory_mem_ck : out std_logic; -- mem_ck memory_mem_ck_n : out std_logic; -- mem_ck_n memory_mem_cke : out std_logic; -- mem_cke memory_mem_cs_n : out std_logic; -- mem_cs_n memory_mem_ras_n : out std_logic; -- mem_ras_n memory_mem_cas_n : out std_logic; -- mem_cas_n memory_mem_we_n : out std_logic; -- mem_we_n memory_mem_reset_n : out std_logic; -- mem_reset_n memory_mem_dq : inout std_logic_vector(31 downto 0) := (others => 'X'); -- mem_dq memory_mem_dqs : inout std_logic_vector(3 downto 0) := (others => 'X'); -- mem_dqs memory_mem_dqs_n : inout std_logic_vector(3 downto 0) := (others => 'X'); -- mem_dqs_n memory_mem_odt : out std_logic; -- mem_odt memory_mem_dm : out std_logic_vector(3 downto 0); -- mem_dm memory_oct_rzqin : in std_logic := 'X'; -- oct_rzqin reset_reset_n : in std_logic := 'X'; -- reset_n mult_data_0_mult_data_m_in1 : out std_logic_vector(31 downto 0); -- m_in1 mult_data_0_mult_data_m_in2 : out std_logic_vector(31 downto 0); -- m_in2 mult_data_0_mult_data_m_result : in std_logic_vector(31 downto 0) := (others => 'X'); -- m_result mult_control_0_mult_control_m_start : out std_logic_vector(31 downto 0); -- m_start mult_control_0_mult_control_m_reset : out std_logic_vector(31 downto 0); -- m_reset mult_control_0_mult_control_m_done : in std_logic_vector(31 downto 0) := (others => 'X'); -- m_done div_control_0_div_control_d_start : out std_logic_vector(31 downto 0); -- d_start div_control_0_div_control_d_reset : out std_logic_vector(31 downto 0); -- d_reset div_control_0_div_control_d_done : in std_logic_vector(31 downto 0) := (others => 'X'); -- d_done div_data_0_div_data_d_denom : out std_logic_vector(31 downto 0); -- d_denom div_data_0_div_data_d_numer : out std_logic_vector(31 downto 0); -- d_numer div_data_0_div_data_d_quot : in std_logic_vector(31 downto 0) := (others => 'X'); -- d_quot div_data_0_div_data_d_rem : in std_logic_vector(31 downto 0) := (others => 'X') -- d_rem ); end component soc_system; component mult_unit is PORT( clk, reset, enable : IN STD_LOGIC; mult_a, mult_b : IN STD_LOGIC_VECTOR(15 DOWNTO 0); mult_done : OUT STD_LOGIC; mult_result : OUT STD_LOGIC_VECTOR(31 DOWNTO 0):= (others => '0') ); end component mult_unit; component div_unit is PORT( clk, reset, enable : IN STD_LOGIC; div_denom, div_numer : IN STD_LOGIC_VECTOR(15 DOWNTO 0); div_done : OUT STD_LOGIC; div_quot, div_rem : OUT STD_LOGIC_VECTOR(15 DOWNTO 0) ); end component div_unit; --SIGNALS instantiated here SIGNAL reset_reset_n : STD_LOGIC; SIGNAL mult_in1, mult_in2, mult_output_result : STD_LOGIC_VECTOR(31 DOWNTO 0); SIGNAL mult_input_start, mult_input_reset : STD_LOGIC_VECTOR(31 DOWNTO 0); SIGNAL mult_done : STD_LOGIC; SIGNAL div_in_d, div_in_n, div_out_q, div_out_r : STD_LOGIC_VECTOR(31 DOWNTO 0); SIGNAL div_input_start, div_input_reset : STD_LOGIC_VECTOR(31 DOWNTO 0); SIGNAL div_done : STD_LOGIC; BEGIN --port map soc_system here u0 : component soc_system port map ( clk_clk => CLOCK_50, reset_reset_n => '1',memory_mem_a => HPS_DDR3_ADDR, memory_mem_ba => HPS_DDR3_BA, memory_mem_ck => HPS_DDR3_CK_P, memory_mem_ck_n => HPS_DDR3_CK_N, memory_mem_cke => HPS_DDR3_CKE, memory_mem_cs_n => HPS_DDR3_CS_N, memory_mem_ras_n => HPS_DDR3_RAS_N, memory_mem_cas_n => HPS_DDR3_CAS_N, memory_mem_we_n => HPS_DDR3_WE_N, memory_mem_reset_n => HPS_DDR3_RESET_N, memory_mem_dq => HPS_DDR3_DQ, memory_mem_dqs => HPS_DDR3_DQS_P, memory_mem_dqs_n => HPS_DDR3_DQS_N, memory_mem_odt => HPS_DDR3_ODT, memory_mem_dm => HPS_DDR3_DM, memory_oct_rzqin => HPS_DDR3_RZQ, hps_io_hps_io_emac1_inst_TX_CLK => HPS_ENET_GTX_CLK, hps_io_hps_io_emac1_inst_TXD0 => HPS_ENET_TX_DATA(0), hps_io_hps_io_emac1_inst_TXD1 => HPS_ENET_TX_DATA(1), hps_io_hps_io_emac1_inst_TXD2 => HPS_ENET_TX_DATA(2), hps_io_hps_io_emac1_inst_TXD3 => HPS_ENET_TX_DATA(3), hps_io_hps_io_emac1_inst_RXD0 => HPS_ENET_RX_DATA(0), hps_io_hps_io_emac1_inst_MDIO => HPS_ENET_MDIO, hps_io_hps_io_emac1_inst_MDC => HPS_ENET_MDC, hps_io_hps_io_emac1_inst_RX_CTL => HPS_ENET_RX_DV, hps_io_hps_io_emac1_inst_TX_CTL => HPS_ENET_TX_EN, hps_io_hps_io_emac1_inst_RX_CLK => HPS_ENET_RX_CLK, hps_io_hps_io_emac1_inst_RXD1 => HPS_ENET_RX_DATA(1), hps_io_hps_io_emac1_inst_RXD2 => HPS_ENET_RX_DATA(2), hps_io_hps_io_emac1_inst_RXD3 => HPS_ENET_RX_DATA(3), hps_io_hps_io_sdio_inst_CMD => HPS_SD_CMD, hps_io_hps_io_sdio_inst_D0 => HPS_SD_DATA(0), hps_io_hps_io_sdio_inst_D1 => HPS_SD_DATA(1), hps_io_hps_io_sdio_inst_CLK => HPS_SD_CLK, hps_io_hps_io_sdio_inst_D2 => HPS_SD_DATA(2), hps_io_hps_io_sdio_inst_D3 => HPS_SD_DATA(3), hps_io_hps_io_usb1_inst_D0 => HPS_USB_DATA(0), hps_io_hps_io_usb1_inst_D1 => HPS_USB_DATA(1), hps_io_hps_io_usb1_inst_D2 => HPS_USB_DATA(2), hps_io_hps_io_usb1_inst_D3 => HPS_USB_DATA(3), hps_io_hps_io_usb1_inst_D4 => HPS_USB_DATA(4), hps_io_hps_io_usb1_inst_D5 => HPS_USB_DATA(5), hps_io_hps_io_usb1_inst_D6 => HPS_USB_DATA(6), hps_io_hps_io_usb1_inst_D7 => HPS_USB_DATA(7), hps_io_hps_io_usb1_inst_CLK => HPS_USB_CLKOUT, hps_io_hps_io_usb1_inst_STP => HPS_USB_STP, hps_io_hps_io_usb1_inst_DIR => HPS_USB_DIR, hps_io_hps_io_usb1_inst_NXT => HPS_USB_NXT, hps_0_h2f_reset_reset_n => reset_reset_n, mult_data_0_mult_data_m_in1 => mult_in1, -- mult_data_0_mult_data.m_in1 mult_data_0_mult_data_m_in2 => mult_in2, -- .m_in2 mult_data_0_mult_data_m_result => mult_output_result, -- .m_result mult_control_0_mult_control_m_start => mult_input_start, -- mult_control_0_mult_control.m_start mult_control_0_mult_control_m_reset => mult_input_reset, -- .m_reset mult_control_0_mult_control_m_done => "0000000000000000000000000000000" & mult_done, -- .m_done div_control_0_div_control_d_start => div_input_start, -- div_control_0_div_control.d_start div_control_0_div_control_d_reset => div_input_reset, -- .d_reset div_control_0_div_control_d_done => "0000000000000000000000000000000" & div_done, -- .d_done div_data_0_div_data_d_denom => div_in_d, -- div_data_0_div_data.d_denom div_data_0_div_data_d_numer => div_in_n, -- .d_numer div_data_0_div_data_d_quot => div_out_q, -- .d_quot div_data_0_div_data_d_rem => div_out_r -- .d_rem ); m0 : component mult_unit port map ( clk => CLOCK_50, reset => mult_input_reset(0), enable => mult_input_start(0), mult_a => mult_in1(15 DOWNTO 0), mult_b => mult_in2(15 DOWNTO 0), mult_done => mult_done, mult_result => mult_output_result ); d0 : component div_unit port map ( clk => CLOCK_50, reset => div_input_reset(0), enable => div_input_start(0), div_denom => div_in_d(15 DOWNTO 0), div_numer => div_in_n(15 DOWNTO 0), div_done => div_done, div_quot => div_out_q(15 DOWNTO 0), div_rem => div_out_r(15 DOWNTO 0) ); End Behaviour;